# TABLE OF CONTENTS

Paper LERG Routing Guide: Beyond Section 1, this Table of Contents is geared towards the “paper” version of the Telcordia® LERG™ Routing Guide.  Sections 1, 2, and 3 are provided in each Volume (1-8 & C) of the LERG Routing Guide.  Each subsection within Sections 4, 5, 6, 7, 8, 9 and 10 indicates the Section and Volume number in which they are contained, e.g., Destination Code Subsection 4.1 will be found in Section Four (4) of Volume 1.  Due to various factors, not all the data in the “data” versions of the LERG Routing Guide is provided in the paper version.

Data LERG Routing Guide:  For the LERG Routing Guide provided on CD ROM and available via web download, the LERGINFO.DOC file contains text regarding only Section 1 of the following Table of Contents.  The other sections noted in the Table of Contents pertain to the paper volumes of the LERG Routing Guide only.  Data contained in the listed “paper” sections appear in various data files on the data versions of the LERG Routing Guide (referred to as LERG1, LERG2, etc.).  The appropriate data files are denoted, when applicable, in the Table of Contents and in the text in Section 1.  Note that there are several files provided in the data versions of the LERG Routing Guide that may not correspond in whole or part to a specific section in this Table of Contents.

LERGINFO.DOC is this document.  Companies receiving the LERG Routing Guide as transmitted over NDM (Network Data Mover, a.k.a. Connect:Direct™) may request a copy by calling the Telcordia Routing Administration (TRA) Customer Service Center on 866-NPA-NXXS (866-672-6997) or 732-699-6700, or may download a copy from www.trainfo.com (documents).

SUBJECT									          PAGE NUMBER

1.	LERG Routing Guide	1

1.1	General	1

1.2	LERG Routing Guide Structure	1

1.3	Glossary	14

1.3.1	State, Province, Island Code Table	53

1.4	Special Routing/NXX Codes	54

1.4.1	Special Plant Test Codes (1XX)	54

1.4.2	Service Codes	55

1.4.3	NANP Universal Central Office Codes ( NXXs )	57

1.4.4	Operator Service Codes	57

1.5	Signaling System 7 (SS7) Network Codes	59

1.6	Vertical Service Codes	60

1.6.1	Introduction	60

1.6.2	Vertical Service Code Assignments	61

1.6.3	Definitions of Vertical Service Code Assignments	63

1.7	Carrier Identification Codes (CICs)	69

1.8	Automatic Number Identification (ANI) Information Indicator (II) Digits Codes	70

1.9	Toll-free (e.g. 800) and 900 NXX Code Assignments, Routing Status, Definitions	75

1.10	500 NXX Personal Communications Services (PCS) Code Assignments, Definitions	77

1.11	710 NXX Code Assignments, Routing Procedures, Definitions	79

1.12	Country Codes	81

1.13	976-LIKE Codes	85

1.14	Telcordia® Routing Administration Product Ordering Information	86

Note: the correlation of LERGx files to a given paper section in the remainder of this Table of Contents is for reference purposes only.  LERGx files often contain more data elements than may exist in the associated paper LERG section.

2.	Routing Contacts and Operating Company Numbers (LERG1, LERG1CON)

This section lists the names, addresses, and telephone numbers of routing contacts sorted by Operating Company Number (OCN).  It also includes OCN assignments sorted alphabetically by OCN name.

  - Listing in Numerical Order

2.2	Listing in Alphabetical Order

3.	NPA, LATA, Signaling System 7 (SS7) Network Code and Country Code Assignments (LERG2, 3, 4, 5)

This section contains the NPA assignments sorted alphabetically by name, NPA assignments listed in numerical order, LATA assignments listed in numerical order, Signaling System 7 (SS7) Network Codes and Country Codes sorted alphabetically by country and numerically by codes assigned.

3.1	Listing Numerically by NPA (LERG3)

3.2	Listing Alphabetically by NPA Name (LERG3)

3.3	Listing of LATAs Numerically within Region (LERG5)

3.4	Signaling System 7 (SS7) Network Code Assignments (LERG4)

3.5	Country Code Assignments (LERG2)

3.5.1	Alphabetical Listing By Country Name

3.5.2	Numeric Listing by Country Code

4.	Destination Codes (LERG6)

This section lists Destination Codes (i.e., NPA NXXs) consisting of an NPA and a Central Office Code (COC, i.e. NXX) in numerical order within each LATA.  It also contains additional information associated with each Destination Code, (e.g., switching entity / POI which often is a COMMON LANGUAGE® Location Identification, CLLIä).  Refer to Section 1.3 for a glossary of terms and symbols.

4.1	Region #1, 100-Series LATAs (formerly NYNEX)

4.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

4.3	Region #3, 300-Series LATAs (formerly AMERITECH)

4.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

4.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

4.6	Region #6, 600-Series LATAs (formerly U S WEST)

4.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

4.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

4.9	Independents, 900-Series LATAs

4. C	Canada, Part of 800-Series LATAs

5.	Switching Entities (LERG7)

This section lists Switching Entities, e.g., End Office, Feature Group B Tandems, Feature Group C Tandems, Feature Group D Tandems, a remote units’ serving Host, etc., alphabetically by switching entity / POI within each LATA.  It also contains information associated with each switching entity / POI, e.g., Vertical and Horizontal Coordinates (VC & HC).  Refer to Section 1.3 for a glossary of terms and symbols.

5.1	Region #1, 100-Series LATAs (formerly NYNEX)

5.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

5.3	Region #3, 300-Series LATAs (formerly AMERITECH)

5.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

5.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

5.6	Region #6, 600-Series LATAs (formerly U S WEST)

5.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

5.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

5.9	Independents, 900-Series LATAs

5.C	Canada, Part of 800-Series LATAs

6.	Rate Centers and Localities (LERG8)

This section lists Rate Centers and their associated Locality abbreviations by LATA.  It also contains additional information associated with each Rate Center, e.g., Rate Center VC & HC.  Refer to Section 1.3 for a glossary of terms and symbols.

6.1	Region #1, 100-Series LATAs (formerly NYNEX)

6.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

6.3	Region #3, 300-Series LATAs (formerly AMERITECH)

6.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

6.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

6.6	Region #6, 600-Series LATAs (formerly U S WEST)

6.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

6.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

6.9	Independents, 900-Series LATAs

6.C	Canada, Part of 800-Series LATAs

7.	Tandem Homing Arrangements (LERG9)

This section lists switches that are “homed” to (e.g. tandems), e.g., Feature Group B, Feature Group C, Feature Group D, Host switches, Operator Services Tandems, Service Switching Point Tandems, 800 SSP Tandems, and their subtending end offices.  This information is sorted alphanumerically in order of LATA, “tandem,” and NPA/COC.  It includes additional information such as status, effective date, switch identification, equipment type, and the originating and termination functions of the tandem.  Refer to Section 7 for complete data field Listing and Section 1.3 for a glossary of terms and symbols.

7.1	Region #1, 100-Series LATAs (formerly NYNEX)

7.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

7.3	Region #3, 300-Series LATAs (formerly AMERITECH)

7.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

7.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

7.6	Region #6, 600-Series LATAs (formerly U S WEST)

7.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

7.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

7.9	Independents, 900-Series LATAs

7.C	Canada, Part of 800-Series LATAs

8.	Operator Services (LERG10)

This section lists operator access codes ordered by NPA/NXX.

8.1	Region #1, 100-Series LATAs (formerly NYNEX)

8.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

8.3	Region #3, 300-Series LATAs (formerly AMERITECH)

8.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

8.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

8.6	Region #6, 600-Series LATAs (formerly U S WEST)

8.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

8.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

8.9	Independents, 900-Series LATAs

8.C	Canada, Part of 800-Series LATAs

9.	Operator Services (LERG11)

This section lists operator access codes listed by state/locality.

9.1	Region #1, 100-Series LATAs (formerly NYNEX)

9.2	Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

9.3	Region #3, 300-Series LATAs (formerly AMERITECH)

9.4	Region #4, 400-Series LATAs (formerly BELLSOUTH)

9.5	Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

9.6	Region #6, 600-Series LATAs (formerly U S WEST)

9.7	Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

9.8	Independents and OFF SHORE, non-Canadian 800-Series LATAs

9.9	Independents, 900-Series LATAs

9.C	Canada, Part of 800-Series LATAs

- Location Routing Number (LERG12)

This section lists Listing of Location Routing Numbers (LRNs) by LATA.

10.1	 Region #1, 100-Series LATAs (formerly NYNEX)

10.2 Region #2, 200-Series LATAs (formerly BELL ATLANTIC)

10.3 Region #3, 300-Series LATAs (formerly AMERITECH)

10.4	 Region #4, 400-Series LATAs (formerly BELLSOUTH)

10.5 Region #5, 500-Series LATAs (formerly SOUTHWESTERN BELL TELEPHONE)

10.6 Region #6, 600-Series LATAs (formerly U S WEST)

10.7 Region #7, 700-Series LATAs (formerly PACIFIC TELESIS GROUP)

10.8 Independents and OFF SHORE, non-Canadian 800-Series LATAs

10.9 Independents, 900-Series LATAs

10.C	Canada, Part of 800-Series LATAs

# 1.	LERG Routing Guide

## 1.1	General

The Telcordia® LERG™ Routing Guide provides routing data obtained from the Telcordia® Business Integrated Routing and Rating System (BIRRDS) into which data is entered by Local Service Providers (LSPs) and/or their agents.  The LERG Routing Guide reflects information contained in the UNDERLYING Telcordia database as of the run date of the LERG Routing Guide production cycle.  With few exceptions, close of business on the last working day of a month is the basis for the LERG Routing Guide dated the first of the following month.

The LERG™ Routing Guide reflects the “current” state of active network data and also reflects “future” activity within the North American Numbering Plan (NANP), as reported by Local Service Providers (LSPs).  “Future” activity should be reported in line with industry notification timeline guidelines, but ultimately is the responsibility of each LSP.  With the exception of NPA (Area Code) split data (that may be entered with a substantial lead time), most changes encompass activity to occur within the next 6 months, although, technically, data can be reported as far in the future as a provider may deem reasonable and permissible within any applicable guidelines that may pertain to the data elements.  It is important to understand that future data will appear in the LERG Routing Guide with an effective date, each month, until the date has passed.  Users of the LERG Routing Guide should understand that future activity (as well as the current network state), as provided in each monthly LERG, is a complete replacement of previously reported activity (i.e. the removal, addition, or change to future information from one monthly LERG Routing Guide to the next is not flagged in the primary main files (e.g. LERG6).  However, files that have associated “insert” files (e.g. LERG6INS) can be used to identify such changes).

The Table of Contents of this document describes the various sections in the Paper LERG Routing Guide.  In data versions of the LERG Routing Guide (CD ROM and available via web download), Section 1 corresponds to the LERGINFO.DOC.  Companies receiving the LERG Routing Guide as transmitted over NDM (Network Data Mover, a.k.a. Connect:Direct™) may request a copy of this document be emailed to them by calling the Telcordia Routing Administration (TRA) Customer Service Center on 866-NPA-NXXS (866-672-6997) or 732-699-6700, or may download a copy from www.trainfo.com (documents).

Please note that any references to “Sections” beyond Section 1 pertain to the paper version of the LERG Routing Guide.  When appropriate, cross-reference is made to LERG Routing Guide data files (e.g. LERG1, LERG2, etc.).

## LERG Routing Guide Structure

The Paper LERG Routing Guide is divided into Sections that consist of Destination Codes (i.e. NPA NXX’s) (Section 4), Switching Entities (Section 5), Rate Centers and Localities (Section 6), Tandem Homing Arrangements (Section 7), Operator Services (Sections 8 and 9), and Location Routing Numbers (Section 10).  These sections are further subdivided by LATA and may be ordered by Regional “volume”, as listed below.  The Data versions of the LERG Routing Guide contain data for the entire NANP (i.e., data for all regions is provided in the data versions of the LERG Routing Guide).  Note that the term “formerly” in the following table refers to former titles of the specific volumes and refers to the Regional Bell Operating Companies (RBOCs) as existed in 1984.

| REGION (VOLUME) | ##### NAME | ##### LATAs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100-Series LATAs (formerly NYNEX) | 120 | 122 | 124 | 126 | 128 |
|  |  | 130 | 132 | 133 | 134 | 136 |
|  |  | 138 | 140 |  |  |  |
| 2 | 200-Series LATAs (formerly BELL ATLANTIC) | 220 | 222 | 224 | 226 | 228 |
|  |  | 230 | 232 | 234 | 236 | 238 |
|  |  | 240 | 242 | 244 | 246 | 248 |
|  |  | 250 | 252 | 254 | 256 |  |
| 3 | 300-Series LATAs (formerly AMERITECH) | 320 | 322 | 324 | 325 | 326 |
|  |  | 328 | 330 | 332 | 334 | 336 |
|  |  | 338 | 340 | 342 | 344 | 346 |
|  |  | 348 | 350 | 352 | 354 | 356 |
|  |  | 358 | 360 | 362 | 364 | 366 |
|  |  | 368 | 370 | 374 | 376 |  |
| 4 | 400-Series LATAs (formerly BELLSOUTH) | 420 | 422 | 424 | 426 | 428 |
|  |  | 430 | 432 | 434 | 436 | 438 |
|  |  | 440 | 442 | 444 | 446 | 448 |
|  |  | 44813 | 44814 | 44815 | 44816 | 450 |
|  |  | 45009 | 45010 | 45011 | 45012 | 452 |
|  |  | 45204 | 45205 | 454 | 45402 | 45403 |
|  |  | 456 | 45601 | 458 | 45806 | 45807 |
|  |  | 45808 | 460 | 46017 | 46018 | 462 |
|  |  | 464 | 466 | 468 | 470 | 472 |
|  |  | 474 | 476 | 477 | 478 | 480 |
|  |  | 482 | 484 | 486 | 488 | 490 |
|  |  | 492 |  |  |  |  |
| 5 | 500-Series LATAs (formerly SOUTHWESTERN BELL) | 520 | 521 | 522 | 524 | 526 |
|  |  | 528 | 530 | 532 | 534 | 536 |
|  |  | 538 | 540 | 542 | 544 | 546 |
|  |  | 548 | 550 | 552 | 554 | 556 |
|  |  | 558 | 560 | 562 | 564 | 566 |
|  |  | 568 | 570 |  |  |  |
| 6 | 600-Series LATAs (formerly US WEST) | 620 | 624 | 626 | 628 | 630 |
|  |  | 632 | 634 | 635 | 636 | 638 |
|  |  | 640 | 644 | 646 | 648 | 650 |
|  |  | 652 | 654 | 656 | 658 | 660 |
|  |  | 664 | 666 | 668 | 670 | 672 |
|  |  | 674 | 676 |  |  |  |
| 7 | 700-Series LATAs (formerly PACIFIC TELESIS GROUP) | 720 720 | 721 | 722 | 724 | 726 |
|  |  | 728 | 730 | 732 | 734 | 736 |
|  |  | 738 | 740 |  |  |  |
| 8 | OFF SHORE AND INTERNATIONAL (e.g. Caribbean NANP, PR, Guam, …) | 820 | 822 | 824 | 826 | 828 |
|  |  | 830 | 832 | 834 | 836 | 870 |
|  |  | 871 | 884 |  |  |  |
| C | CANADA (entire country) | 888 |  |  |  |  |
| 9 | INDEPENDENTS (certain non-RBOC areas with U.S.) | 920 | 921 | 922 | 923 | 924 |
|  |  | 927 | 928 | 929 | 930 | 932 |
|  |  | 937 | 938 | 939 | 949 | 951 |
|  |  | 952 | 953 | 956 | 958 | 960 |
|  |  | 961 | 963 | 973 | 974 | 976 |
|  |  | 977 | 978 | 980 | 981 |  |

LERG Routing Guide FORMAT

- General formats of the Paper LERG Routing Guide:

- Sections are numbered consecutively beginning with Page 1 within each section.

- Section Title appears in the top center of each page.

- The paper version of the LERG Routing Guide does not contain all the data files and/or data elements in specific files that may appear in the more comprehensive data versions of the LERG Routing Guide.

- General formats of the Data LERG Routing Guide:

- Generic:

- LERG Routing Guide “files” are numbered (e.g. LERG1 = OCN Information), with files containing the same number, containing variations or extensions of closely related data (e.g. LERG8 and LERG8LOC).

- Specific file layouts of the LERG flat files are provided in the LERGSPEC.DOC file.  Copies of this document can be also downloaded from www.trainfo.com (documents).

- Special License Versions (SLVs) of the LERG (data only) may, at times, be provided in lieu of the “LERG”.  However, any files that are also provided in such SLV LERGs that are named per files in this document are structured consistent with the descriptions noted herein.

- The LERG One-Day Changes Process involves a file that does not correlate in name and layout to a monthly LERG; files that are provided daily are also a subset of all the files provided in the LERG.  For further information, see the LERG One-Day Changes Process section of this document.

Microsoft Access Version of  LERG data:

- In addition to the LERG, files described above, data on the CD ROM, and as can be downloaded from the web, also contains a Microsoft Accessâ database version of the LERG data.  This is an alternate view of the data in the flat files.  The data in the flat files and in the database files are identical for a given month.  However, please note that Microsoft Access data files (e.g. LERG1 as provided in the Access database) may have data in datasheet view provided in a different sequential and/or sort order than described in LERGSPEC.DOC.  Column headings in the Microsoft Access files may also differ slightly from the wording in LERGSPEC.DOC and from the paper version of the LERG Routing Guide.  Filler space is not shown in the Access tables.

- The Microsoft Access database version of the LERG provides a graphical user interface front end that can be used to access and view data records using a TRA designed "form view" as well as Microsoft Access "table views".  The database requires users to have a local licensed copy of Access.  Users with a basic knowledge of Microsoft Access can easily search for specific data elements, create customized queries, cross-relate data files, export data to other software (e.g. Microsoft Excel), generate files that can  serve as input files downstream operations support systems, etc.

LERG Routing Guide DATA FILE OVERVIEW

The following provides a synopsis of the “data” files provided in the LERG.  Additional information can be found in the GLOSSARY included in this document and well as in the LERGSPEC.DOC file that provides data file layout information.

Most LERG files have a few common elements that can be used to “tie” them together to permit integrated reports, analysis, etc.  Those obtaining the LERG other than via NDM have a Microsoft® Access version of the LERG data (in addition to a “data” (flat file) version).  Microsoft® Access features provide an ability to “tie” (link, etc.) appropriate files together.

LERG1 (Operating Company Number (OCN) information)

The LERG1 file provides high-level information about Operating Company Numbers (OCNs) such as name and type of company.  This includes the contact information (for purposes of LERG data) as provided to TRA.  Note that contact information is maintained as best as possible; however, due to constant changes in personnel in a company, reorganizations, mergers, acquisitions, etc., some information will not be up-to-date.  In addition, the contact noted may not necessarily be familiar with all aspects of a company’s operation (network, billing, etc).  The contact name defaults to the party requesting the assignment of the number used as the OCN unless the company has specifically requested a given party be listed for purposes of addressing LERG data.

LERG1CON (additional “contact” information for OCNs)

The LERG1CON file is information directly entered by each OCN or its agent.  All OCNs will have, at minimum, a SERVICE OF SUBPEONA contact identified (there may not necessarily be a telephone or additional information though).  Please note that since these records are established on a per company basis, the extent of information may vary.  Maintenance of the data is the purview of the OCN.

LERG2 (Country Code information)

The LERG2 file is independent from all other LERG Routing Guide files.  This file contains high-level country Code information (e.g. Republic of Hungary = 36).  There is currently no City Code information in this file.  The file is provided for your reference in a data format.

LERG3 (NPA (area Code) information)

The LERG3 file contains high-level information about NPAs (Area Codes).  This includes the date the NPA went or is going into effect, permissive dialing periods for splits, the NPA that had previously served the area (or continues to do so in a case of an overlay), etc.

LERG4 (SS7 “Point Code” assignments to companies)

The LERG4 file contains SS7 “point code” assignments at the network, cluster, or member levels of coding.  Network and cluster code assignments provide for a company to assign member level codes on its own.  Level of assignment is based on a combination of need/request as well as the specifics about a given company’s network.  This file simply identifies the company to which the code has been assigned; it does not associate any network elements to any specific point codes.

LERG5 (NPA/LATA cross reference)

The LERG5 file identifies the NPAs in use within a LATA.  The LATA information is then grouped by Region.

LERG6 (NPA NXX BLOCK information)

The LERG6 file NPA NXX (central office code) information and where applicable, BLOCK assignments within the NPA NXX.  NPA NXX assignments (those made by the CO Code Administrator) are represented with a BLOCK value of “A”.  Thousands Block Pooling assignments (those made by the Pooling Administrator) are represented by numeric BLOCK values (0-9).  Only those BLOCKS actually assigned are provided (i.e., not all blocks 0-9, may appear).  This file includes among other data, the Rate Center associated with each record, the OCN (company assigned the record), the serving switch/POI with an associated switch homing arrangement (SHA) Indicator, the identified “use” of the record (e.g. cellular, wireline … See COCTYPE/SSC), and LATA (both switch and Rate Center).

Note regarding “A” BLOCKs and numeric BLOCKs:

- A designated CO Code Administrator in each NANPA country assigns NPA NXX’s.  An “A” in the BLOCK field of LERG6 files identifies such assignments when the NXX begins with a 2-9.  In many cases, numeric BLOCKs will appear associated with an NPA NXX.  Numeric blocks are assigned by the Pooling Administrator (as of December 1, 2003 this applies only to the United States except for Alaska, Guam, the Commonwealth of the Northern Marianas, and the U.S. Virgin Islands).  Information provided for numeric BLOCKs (e.g. COCTYPE, OCN, …) can be viewed as an override to the associated “A” BLOCK information.  The “A” BLOCK information exists primarily for default routing purposes.  In cases where no numeric BLOCKs exist for an NXX, it is assumed that the “A” BLOCK information pertains to all telephone numbers associated with the identified CO Code Assignee.

Notes regarding “Portability”:

- Service provider “portability”, that is, the ability for subscribers to change local service providers without having to change their telephone numbers, began in the late 1990’s for the wireline segment of the telecommunications industry, and in November 2003, for the cellular segment of the industry.  The LERG does not provide line-level information, i.e. mapping each telephone number that may be ported to the specific carrier involved.  The company associated with an “A” BLOCK and, where effected, to numeric BLOCKs, is the company assigned the numbering resource by the appropriate administrator (see above).  Only that company may assign numbers to subscribers from vacant numbers within the implied line range(s).  Once assigned, the subscriber may opt to “port” the number to another subscriber.

Such service provider portability does not equate to location/geographic portability.  As has been the case for decades in many areas, wireline subscribers can retain their telephone number ONLY if they remain (to be rated) within Rate Center boundaries – this concept is not changed under “portability”.  Although wireless subscribers can live temporarily or permanently outside the designated Rate Center area associated with the NXX and, depending on their rate plan, not incur any additional charges, those “calling” the subscriber will have their call treated as if they were calling the NXX’s designated Rate Center.  Such a situation may result in the caller having to pay (or not pay) toll charges, depending on their rate plan.

- The term “portability” can be misinterpreted when not qualified.  The routing portability aspects of Local Number “Portability” are a requirement for Thousands Block Pooling and are used by some companies as a means of Intra-SP porting (moving numbers within a company to alternate switches).  However, both these cases do NOT, of and by themselves, imply service provider portability, as previously defined, also applies.  The term “portability” is often used to mean various different things.  The ambiguity has led to some degree of confusion in both providing and interpreting data, policies, etc., throughout the industry.

- The LERG does provide some information regarding portability, identifying NXX’s in which ported numbers may exist, and provides additional data in support of portability such as Location Routing Numbers (LRNs).  Many aspects of “routing” a call, a primary use of the LERG, do not change, and in fact, can support portability.

LERG6ATC (ATC information)

The LERG6ATC file is simply an expansion of data for those records in the LERG6 file that have a COCTYPE value of ATC (Access Tandem Code).  This file expands the information by appending the Operator Service “service” codes for each ATC record.  Please note that the NPA NXX information for ATCs (besides the OS codes) is also included in the primary LERG6 file.  ATC records will not have numeric blocks associated with them.

The LERG6ATC file is only provided in the data versions of the LERG.

LERG6ODD (expanded information for ODDBALL NPA NXX’s)

The LERG6ODD file is simply an expansion of data for those records in the LERG6 file that have COCTYPE values designated as “oddball.”  “Oddball Codes” generally do not have a Rate center or switch identified with them.  These codes exist for various reasons and include generally defined NXX’s such as N11s, 958 (testing), 976 (Information Providers), as well as NXX’s that may be established by a given company, within a given NPA, etc. for various extraordinary reasons and services.  This “Oddball Code” file lists a “notes” field for any ODDBALL NPA NXX record that may have the “notes” field populated.  Please note that the NPA NXX information for oddball codes (besides the Notes) is also included in the primary LERG6 file.

The LERG6ODD file is only provided in the data versions of the LERG.

LERG6INS (month-to-month changes)

The LERG6INS file, as well as all other “insert” files in the LERG, represents data “changes” between the current monthly LERG and the LERG issued the previous month.  The data in the “insert” file is included in the primary file (e.g. LERG6) as well.  Thus, use of this file can be considered “optional” depending on a LERG user’s desire and need in reconciling data changes appearing in the primary files and/or in tracking month-to-month record changes.  Since the monthly LERG is based on data that has been provided to TRA on the last business day of a month (e.g. end of March for the April 1 LERG), the database underlying the LERG may be updated immediately thereafter.  The “insert” file represents changes from the “close” of the previous LERG.

- Changes made since the previous LERG and effective in the prior month are included in the “insert” file with an appropriate EFFDATE that is after the close of the previous LERG.  These cases will appear in the current primary LERG file as “current” (blank EFFDATE and STATUS).  Disconnects falling under this scenario will not appear in the primary file.

- Other changes in the “insert” file constitute “future” views that, depending on the indicated activity (add/remove), would correspondingly have, unless removed, the same future effective date in both the “insert” and primary files.  “Changes” are based on EFFDATE and can involve an EFFDATE view that was added, removed, or was previously issued but has one or more associated data field values changed.  Only future activity that was changed, removed, or added since the last LERG will appear in the “insert” file, NOT all future activity.

- Note that data changes that only impact the AOCN field are not included in the “insert” files.

For additional information, see “Insert Files” in the Glossary.  “Insert” files pertain only to the data versions of the LERG.

LERG7 (Switch/POI information)

The LERG7 file contains information regarding “switches.”  Such “switches” are identified by an 11-character code that, with few exceptions, should be a CLLI™ code as are trademarked and supported by the COMMON LANGUAGE organization in Telcordia Technologies, Inc.  Although often termed a “switch,” there may be multiple CLLI codes for a single physical switch for various reasons.  Such reasons can include the switch performing multiple functions (e.g. as an end office and as a tandem).  CLLIs may also reflect a “Point of Interface (POI)” established as the interconnection point between two carriers.  Although POIs may, at times, be at the same location as an actual physical switch, they do not have to be.  All assigned CLLIs are not listed in the LERG – only those that are relevant to accessing the local network and routing calls within the local network are included.  This file provides high-level switch information such as address, V&H coordinates, switch functionalities, equipment type, etc.

LERG7INS (month-to-month data changes)

See the LERG6INS description.

LERG7SHA (Switch homing arrangement information)

The LERG7SHA file contains information regarding the ‘homing” arrangements associated with a given “switch.”  Homing arrangements include such situations as the Feature Group B, C, and D tandems associated with a switch; STPs; in the case of remotes, their Hosts; in the case of Points of Interface, their ACTUAL SWITCH; etc.  This file expands upon the higher-level information about a “switch” that is provided in LERG7.

Note that the SHA IND field is critical in assessing homing arrangements and their application against other LERG data.  A given “switch” may have more than one Feature Group B, C, or D tandem, etc., associated with it.  This is due to various factors.  If/when attempting to identify a specific tandem for a given NXX in LERG6, the “switch” and associated SHA IND in LERG6 must be tied in unison to the same combination in LERG7SHA to determine the appropriate homing.

The LERG7SHA file is only provided in the data versions of the LERG.

LERG7SHI (month-to-month data changes)

See the LERG6INS description.

LERG8 (Rate Center information)

The LERG8 file represents Rate Centers (also known as Exchange Areas).  These are geographical areas defined historically and ongoing based on various factors that can vary by state/province.  “Boundaries” of a Rate Center may be a section of a large city, boundaries of a specific town, or an area that may encompass multiple towns.  Rate center “boundaries” are within state/province boundaries; however, correlation with other factors such as geographical limitations (e.g. rivers), town boundaries, county boundaries, etc., can exist, but are not requirements (these factors may vary by state).  In general, Rate Centers historically define an area in which wireline (or wireline to cellular) calls that have originated and terminated within its boundaries (based on NXX’s defined to the Rate Center) are considered local (non-toll) calls.  However, note that local calls can also extend to other (e.g. adjoining) Rate Centers (information regarding this extension of local calling area is not provided in the LERG).

Rate Centers are generally defined in tariffs filed by principal wireline carriers in the area and are followed by companies that do not need to file tariffs, due to the potential issues surrounding billing that may otherwise occur.  Rate Centers also have associated V&H Coordinates, derived from Latitude and Longitude, which are then used to compute distances between Rate Centers.  Such distances are a component in determining charges for a call for rate plans that are based on distances.

LERG8LOC (Localities associated with Rate Centers)

The LERG8LOC file identifies “localities” associated with a given Rate Center (LERG8).  Often, several “localities” (e.g. towns) are included in a Rate Center.  This information can be useful in cases where a town is known and its Rate Center needs to be identified.  Note that the localities identified for a given Rate Center are not intended to be a complete listing of each and every defined jurisdiction, alternative name, jurisdictional subsection, etc. that exists within the Rate Center; however, the majority of major localities should be identified.

The LERG8LOC file is only provided in the data versions of the LERG

LERG9 (Homing Arrangements by homed-to switch)

The LERG9 file is a “processed” file in that the data contained in it is a processing of data provided in LERG6, LERG7, and LERG7SHA and does not contain any information beyond what is in those files.  LERG9 essentially assesses the “homing” of switches provided in LERG7SHA and summaries that information by homed-to switch (e.g. by STP, Tandem, etc.).  In addition, associated NPA NXX information (based on “A” BLOCK records only) is drawn from LERG6 data.  Some additional LERG6 field values and some from LERG7 are also included.

Homing arrangements provided in LERG9 pertain to the “A” BLOCK records in LERG6, not the numeric blocks.  Numeric blocks require routing portability to exit.  Such routing is based on the Location Routing Number (LRN) as may be associated with each individually ported telephone number that resides in the Number Portability Administration Center’s (NPAC’s) Service Management System (SMS) database.  LRNs can essentially be viewed as telephone numbers on which call set up and routing occur, thus LERG9 can be used to “route” LRNs, as if the LRN were an NPA NXX.

This file provides a “top down” view of homing arrangements.  For example, it provides the NPA NXX and switches that may subtend a given Feature Group D Access Tandem, a Local Tandem, a Host, etc.  This information is also discernable from LERG7SHA as well.

LERG9 has historically been the largest file in the LERG, primarily due to all the various combinations of data that are possible.  For example, all NPA NXX’s for a switch may be replicated under multiple homed-to switches (e.g. Operator Services, Feature Groups B, C, D, etc.) depending on how many switches a given switch may “home to” in LERG7SHA.

Since the data is, in essence, provided in other LERG files, users should consider whether they should use LERG9 or LERG6, LERG7, and/or LERG7SHA, depending on what information is required.

LERG10

(To be expanded upon at a later date.)  This file provides information derived from manipulation of data in other LERG files.  Its primary purpose is in support of Operator-to-Operator interactions.  Data is specifically associated with Access Tandem Codes (ATCs) interrelationships.

LERG11

(To be expanded upon at a later date.)  This file provides information derived from manipulation of data in other LERG files.  Its primary purpose is in support of Operator-to-Operator interactions.  Data is specifically associated with Access Tandem Codes (ATCs) interrelationships.

LERG12

The LERG12 file is a means by which companies are to report the Location Routing Numbers (LRNs) that are or will be used in relationship with numbers they intended to route via Locality Number Portability routing concepts.  This file identifies the company that has established the LRN as well as the switch to which that LRN should be associated.

LERG13

There is currently no LERG13 file.  A LERG13 file existed for several years.  Beginning with the December 1, 2003 LERG, the then existing LERG13 file was renamed to LERG6, and the then existing LERG6 file format was eliminated.

LERG14

This file provides data providers an opportunity to identify the tandem to which specific NPA NXX BLOCKs can be routed.  This would apply to cases where the tandem provider is not the NPA NXX BLOCK assignee.  In a similar manner, this file can be used by those who chose to enter data, as a means to identify where specific “Oddball” NXX’s that may be associated with an OCN of MULT (multiple companies) should be routed if other need to route such calls to them.  For example, (NPA) 555-1212 calls (which are handled differently by each carrier in a given area).

The LERG14 file is only provided in the data versions of the LERG

LERG15

The LERG15 file provides, for those companies who have supplied data, a relationship between 911 Tandems and the counties (or county equivalents such as parishes in Louisiana) that the Tandem serves.  The Tandem may not necessarily serve the entire county; however, at least some area with the county is served.

The LERG15 file is only provided in the data versions of the LERG

LERGEND

This file provides information regarding the date of a particular LERG; identifies the data “cut-off” date used to extract data from the underlying database; and provides the record counts in each LERG data file.

The LERGEND file is only provided in the data versions of the LERG

LERG Routing Guide PRODUCTION

The LERG Routing Guide is a “snapshot” of a database that is being continuously updated.  The database is downloaded during the evening of the last working day of each month to produce the LERG Routing Guide that is dated the first of the following month.  Slight variations to the last workday scenario may occur at the end of May and December, depending on when Memorial Day and New Year’s Day fall on the calendar, as well as on rare occasions due to other factors.

The LERG Routing Guide is provided in three different manners.  The media and their publication schedule are as follows:

- LERG Routing Guide BOOK (paper report format)

- 8 Volume Set - By LATA grouping, plus a separate volume for Canadian data

Produced Quarterly

- NDM (Network Data Mover) (requires dedicated line and NDM software)

- Data for entire NANP area (dat files only)

Produced Monthly

- CD ROM (data format)*

- Data for entire NANP area

Produced Monthly

* The files provided on CD ROM can also be alternatively downloaded from a web site provided by TRA.

Note:

Since the underlying LERG database is updated by data providers on a daily basis, there is also a “LERG One-Day Changes Process”, further explained in the next subsection of this document.

- LERG Routing Guide ONE-DAY CHANGES PROCESS

Subscribers to the monthly LERG Routing Guide data may optionally subscribe to a LERG Routing Guide “One-Day Changes” Process.  This process assesses daily (calendar day) activity in the underlying database to the LERG Routing Guide and provides resulting “changes” in a format consistent with the associated LERG Routing Guide data files that are addressed as part of the process.  Five LERG data files are currently addressed by this process (LERG1, LERG6, LERG7, LERG7SHA, and LERG12).

Subscribers to the LERG One-Day Changes Process have access to a “LERG One-Day Changes Process Overview” document that provides more details.  Note that the LERG One-Day Changes Process includes an additional file, LERG67, which is unique to the process.  Further information on LERG67 is also provided in the Overview document.

The LERG One-Day Changes Process identifies “changes” (i.e., new records, changed data, etc.) in a data format.  The process does NOT generate a complete file (for the files involved); it only provides information about records that had a change activity.  The process does NOT directly “update” the LERG Routing Guide files provided by TRA.  Use and incorporation of these change files into internal user processes is the responsibility of each user.

- For further information, please call the Telcordia Routing Administration (TRA) Customer Service Center on 866-NPA-NXXS (866-672-6997) or 732-699-6700

## 1.3	Glossary

The terms, data values, etc., described in this Glossary may pertain to one or more sections (and/or files) of the LERG Routing Guide.  Some descriptions are the result of a consensus process through which data providers and recipients have reviewed and concurred with descriptions.  In some cases, the nature of the industry is such that a specific description must be kept at a high level to accommodate differences in interpretations, service naming, etc.  For the most part, data fields listed in the GLOSSARY are ordered based on their identification as fields in the LERG files as provided in LERGSPEC.DOC.

Notes:

- References to files in this glossary that are “associated” with a “primary” file (e.g. LERG6INS, LERG6ATC, etc. are associated with LERG6) are implied by noting the primary filename.  Such files either have very similar file structures, or else the data is closely related.

- Terms listed below are not all just “fields” identified in the LERG.  Some appear for information and clarification purposes.

- “Fieldnames” in the Microsoft Access database version of the LERG and references to those fields in the LERGSPEC.DOC file may vary slightly for various, primarily cosmetic, reasons.

- Terminology used in the telecommunications industry to describe a situation, data element, service, etc. often varies among service providers, regulatory bodies, subscribers, etc.  In other cases, definitions may change over time to address clarifications, new technology, etc. and may not be carried through to all uses of terms.  Please be aware of this in relating LERG data to your needs, data requests, etc.  In many cases, there is no one definitive, maintained, and all-inclusive source for definition of telecommunications terms.

| ACTION | — | For NPA data, this will indicate an S (split) or O (overlay) that has been taken to address NPA relief needs. (LERG3, Section 3.1) |  |  |
| --- | --- | --- | --- | --- |
| ACTIVITY CODE | — | Indicates the activity associated with the transaction being report in the Insert file as indicated below. Can be viewed as indicating what action should be taken regarding the transaction. This is similar to this field as used in the LERG One-Day Changes process, from which this data in the Insert File is generated. (Insert files) (See also Insert Files, and DATE OF ACTIVITY) A = “Add” - An indication of a newly reported (“added”) line. This information was not previously reported, or was reported as “R” the previous day or earlier, and has been added back. R = “Remove” - An indication that the information is to be “removed” at the level of uniqueness. This could be the result of a canceled or rescheduled activity. U = “Update” - An indication of an “updated” line. Updates are modifications to records at the level of uniqueness (e.g. EFFDATE) THAT WERE PREVIOUSLY REPORTED (either in the previous monthly LERG or with an earlier Date of Activity in the given “Insert” file) but for which data changed I = “Informational” – An indication of changes that were effective on or before the Activity Date. This most often will apply to certain fields, such as AOCN, that are not date-dependent but can change values. “I” Activity Codes (as with “A” and “U”), as reported in the “Insert” files, do represent changes that were not previously reported and should be treated as such by those using these files. A record with an “I” Activity Code will always have its Activity Date equal to its EFFDATE and will always reflect dates within the past two months from the LERG month. NOTE: In the One-Day LERG Changes process, records with an ACTIVITY CODE of “I” indicate that the “current” state of the data has changed from one day to the next. This can occur for two reasons: 1) To indicate that a previously reported intention of change has now become effective. In this case, these lines are not the result of a newly entered change to the data. |  |  |
|  |  | 2) In some cases, data changes are entered to the BIRRDS database with an immediately EFFDATE. As a consequence, the LERG One-Day Changes process has no advance information to have passed on at any earlier date and an “I” ACTIVITY CODE is generated and is the only advice you will receive of the change. “I” ACTIVITY CODE - LERG INSERT Files: “I” Activity Code records serve an analogous purpose in the LERG Insert files. However, the first situation noted above does not apply to the Insert files; the second situation though, serves a substantive need and is included in the Insert files. As a result of nuances and anomalies in data processing, a few cases of records due to the first situation cited above may appear in the Insert files. This should not be an issue in that it is reiteration of information previously reported. |  |  |
| ACTN DATE | — | This field is in the Country Code (LERG2) data file ACTN DATE is the date associated with the last “action” (ACTN DESC) relative to the data associated with the Country Code. |  |  |
| ACTN DESC | — | This field is in the Country Code (LERG2) data file E=Effective — This is the date the country code has or will become effective. D=Delete — This is the date the country code was discontinued (unassigned) M=Modify — This is the date some information associated with the country code last changed ACTN DESC is provided in conjunction with ACTN DATE. Past dates will appear. In cases where country codes have been existence for more than the past few years, and there have been no modifications over that time, the ACTN DATE and ACTN DESC fields will be blank. |  |  |
| ACTUAL SWITCH ID | — | The 11-character switch identifier of an actual switching entity. The ACTUAL SWITCH is populated to identify the actual switch for a switch identifier that is not a physical switch but rather is a Point of Interface (POI) to which an NPA NXX BLOCK has been, or may be, assigned. The ACTUAL SWITCH may or may not be in the same LATA (or state/province) as some, or all, of its related POIs. (LERG7SHA) |  |  |
| ANI II Digit Codes | — | Automatic Number Identification Information Indicator Digit Codes. These are two-digit codes that precede the 7 or 10-digit directory number (DN) of the calling line. It informs carriers about the type of line that is originating the call, any special characteristics of the billing number, and/or identifies certain service classes. The two-digit codes and the directory numbers are part of the signaling protocol in equal access offices and are outpulsed by the originating switching system to the receiving office for billing, routing, or special handling purposes. |  |  |
| AOCN | — | Administrative Operating Company Number. This identifies the company responsible for the maintenance of a particular record in the database underlying the LERG Routing Guide. If the record administrator has an OCN (Operating Company Number) assignment based on a NECA assigned Company Code, that OCN (or one chosen should multiple Company Code Assignments exist) is generally used as an AOCN value. If the record administrator does not have a NECA assigned Company Code, or chooses not to use it as an AOCN value, TRA assigns a unique OCN for this purpose. (See also OCN; appears in various files and reports sections – refer to OCN for the actual name of the AOCN). |  |  |
| ASSIGNEE COMPANY | — | For SS7 assignments, this is the name of the company that has been assigned the resource at the given level of assignment. (LERG4, Section 3.4) |  |  |
| ATC | — | Access Tandem Code - A three digit “NXX/COC” code in the 0/1XX range that uniquely identifies a tandem providing Local Service Provider operator and/or testing access, or a “+” symbol that indicates direct routing to the designated switch in the NPA. (See COCTYPE.) Note that ATC is NOT intended to signify Feature Group B/D tandems that are also often termed “access tandems.” |  |  |
| BASIS FOR NPA | — | In providing historical NPA information, this indicates the NPA(s) that served as the basis for (immediately preceded) the given NPA in an area. In cases of overlays, some/all of this NPA(s) may still cover the area. (LERG3) |  |  |
| BCR5 | — | An Integrated Service Digital Network (ISDN) Basic Rate Interface (BRI) access capability that allows a customer premise device to communicate directly with the network and/or another ISDN equipped location utilizing an out-of-band signaling protocol and has a data rate of 56Kbps. BRI is two bearer channels, which can be used for voice and data, and one data channel that is used for signaling (2B+D). (SOF Indicator) |  |  |
| BCR6 | — | An Integrated Service Digital Network (ISDN) Basic Rate Interface (BRI) access capability that allows a customer premise device to communicate directly with the network and/or another ISDN equipped location utilizing an out-of-band signaling protocol and has data rates of 56Kbps or 64Kbps clear. BRI is two bearer channels, which can be used for voice and data, and one data channel that is used for signaling (2B+D). (SOF Indicator) |  |  |
| BILL-TO RAO | — | See RAO. (LERG7) |  |  |
| BIRRDS | — | The Telcordia® Business Integrated Routing and Rating System |  |  |
| BLOCK ID | — | In LERG6 files, this will contain an “A” for NPA NXX records that are “assigned” to the Code Holder per the Central Office Code Assignment Guidelines. Various NPA NXX records in the LERG6 files will, in addition to the “A” record, have one or more numeric BLOCK IDs (0-9) identified, as per the Thousands Block Pooling Administrative Guidelines. In the case of numeric BLOCK IDs, this correlates to the 1000 line numbers that begin with the BLOCK ID “thousand” (e.g. BLOCK ID 3 correlates to a range of 3000-3999). Note: A “full set” of BLOCK IDs (i.e., 0-9 all inclusive) should not be assumed in LERG6 files – only those assigned as a ‘pooled range’ or involve NXX’s otherwise ‘split’ and identified by the data provider will be provided. Note: ATC codes will never have a numeric block and “oddball” codes for the most part should not. (See TBP IND, BLOCK ID) (LERG6) |  |  |
| CALL AGENT | — | A call agent switch provides program and call control to manage distributed high performance network gateway equipment (LERG7, LERG7SHA) |  |  |
| CAP | — | Competitive Access Provider - A carrier that provides wireline non-switched access services as an alternative to a local exchange carrier. Also may be referred to as an Alternative Access Provider (AAP) or by other terms. (See CATEGORY) |  |  |
| CATEGORY | — | Identifies the “type” of company associated with a Company Code / OCN. (LERG1, Section 2.2): CAP = Competitive Access Provider CLEC = Competitive Local Exchange Carrier GENERAL = Various miscellaneous cases INTL = International L RESELLER = Local Reseller IC = Interexchange Carrier ICO = Independent Telephone Company INTRALATA = IntraLATA Service Provider P RESELLER = Personal Communication Services Reseller PCS = Personal Communications Service RBOC = Regional Bell Operating Company ULEC = Unbundled Local Exchange Carrier WIRELESS = Wireless Provider (Non-PCS Cellular, Paging, Radio) W RESELLER = Wireless Reseller Note: The term “Incumbent” correlates to ICO and RBOC combined. |  |  |
| CCS AC OFC | — | A switch whose functionality is that of a switch in the CCS network with Access Capability as outlined in TR-394. (SOF Indicator) A switching entity with this functionality must have its Signaling Transfer Point fields (STP1 and STP2) populated. An access purchaser can use common channel signaling to a CCS AC OFC. A switching entity without this functionality may have its STP1 and STP2 fields populated if it is capable of IntraLATA common channel signaling. An access purchaser cannot use common channel signaling to an office that is not a CCS AC OFC. |  |  |
| Central Office Code | — | See COC |  |  |
| CHANGE SOURCE | — | The “Change Source” field assists in isolating the “source” of a given change (in conjunction with the STATUS field) in the homing arrangements. The following values will appear in this field whenever a STATUS value is non-blank: (LERG9, Section 7) Value Description D Destination Code activity only, such as an establishing or disconnecting code or non-switch modification. L Destination Code activity only, where an existing code is moving to a new switch. S Switch activity only, such as the establishment or disconnecting of a switch, or non-homing modification. H Switch Homing activity only, where an existing switch is being re-homed from one tandem to another. B Cases where Destination Code activity (e.g. D,L) and switch activity (e.g. S,H) occur on the same effective date. T Changes to tandem level data (e.g. establishment, disconnect, or data modification). |  |  |
| CIP | — | Carrier Identification Parameter is a special option (SOF Indicator) that the switch is to transmit the three (3) or four (4) digit CIC of the presubscribed line back to the customer. (SOF Indicator) |  |  |
| CITY | — | City name portion of switching entity address. (various files and reports) |  |  |
| CLASS 4/5 | — | A switching entity that performs both a Class 4 and Class 5 function. The CLASS 4/5 office is a single processor switching entity that provides line side and trunk/toll side capabilities to its end users. (SOF Indicator, LERG7) The Class 4 function allows the switching entity to perform Tandem type functions, which may include FG B/C/D assess service, and data base query functions, Operator Services functions, etc. It also provides access on a toll basis to all subtending offices below the Class 4 office including Host/Remote arrangements. |  |  |
|  |  | The Class 5 function allows the switching entity to perform at the lowest level of switching with the LEC network. This function allows end users to receive dial tone; pass digits for call routing; provide line-side features such as call waiting, call forwarding, etc.; and provides Telephone Number association for terminating calls. There should be two switch identifiers associated with this switching entity, one for the class 5 functions and one for the class 4 functions with the class 5 switching entity homing on the class 4 switching entity. |  |  |
| CLEC | — | Competitive Local Exchange Carrier - A wireline based local exchange (switched and non-switched) carrier serving in a geographical area that is already served by an incumbent local exchange carrier (RBOC or ICO). Also referred to by other names including Alternative Local Exchange Carrier (ALEC), Other Local Exchange Carrier (OLEC), Alternate Exchange Carrier (AEC), etc. (See CATEGORY) |  |  |
| CLN INDICATOR | — | This indicator notes whether the SWITCH value is or is not a valid CLLI (by nature of its being in the Telcordia CLONES™ database) on the day before the LERG data was generated (Note: the LERG data is generally generated at close of business on the last business day each month). (LERG7) A = In CLONES, Active I = In CLONES, Inactive D = In CLONES, Delete N = Not in CLONES R = Was in CLONES, but was removed at some point (note: tracking of this began 8/04, earlier cases of records removed from CLONES would appear as N) Note: A, I, and D are consistent with similar indicators in CLONES; N and R are generated for the LERG for reference. |  |  |
| CLUSTER | — | Generally viewed as the middle of the three sets of three numbers comprising a point code (NETWORK-CLUSTER-MEMBER) (LERG4) |  |  |
| CNTY (COUNTY) | — | This is a two-alpha abbreviation that denotes the county in which the associated record resides. This is completed only when more than one record using the same identifying keys exists within a state, otherwise the field is blank. (LERG6, LERG8, Sections 4,6) |  |  |
| COC | — | Central Office Codes (COC), essentially an NXX, also may be referred to as a Destination Code (various files and sections) Codes 0/1XX are used for operator access tandem codes and testboard addressing or a “+” symbol that indicates direct routing to the designated operator switch in the given NPA. 2XX-9XX values are considered NXX’s. (See ATC) |  |  |
| COC TYPE | — | A three-digit code defining the use of the Central Office Code. (LERG6, LERG9, Sections 4,7) ATC = Access Tandem Code (0/1XX) EOC = End Office Code PLN = Planned Code - non-routable PMC = Public Mobile Carrier (Type 2 Interconnected) RCC = Radio Common Carrier (Dedicated Type 1 Interconnected) SIC = Special 800 Service Code SP1 = Service Provider - Miscellaneous Service (Type 1 Interconnected) SP2 = Service Provider - Miscellaneous Service (Type 2 Interconnected) TST = Standard Plant Test Code |  |  |
|  |  | The following are considered COC TYPE values for ‘ODDBALL’ Codes and appear in LERG6 files and, rarely, in LERG9. Oddball Codes often are not associated with a specific LATA (99999 used as a default), Rate Center (XXXXXXXXXX), and/or switch (XXXXXXXXXXX). In many cases, the NXX is not associated within a single OCN (e.g. 911) in which case an OCN value of MULT is used. AIN Advanced Intelligent Network -Usually a routing code (NXX) assigned to a Local Service Provider (LSP)offering a wide range of circuit switched services and utilizing elements of Advanced Intelligent Network (AIN) platforms to determine proper termination of a call. AIN codes may terminate to different locations dependent upon where the call originates, and requires a database application and/or query for processing BLG Billing Only - A non-ratable NXX used for billing purposes only. These codes are used to generate bills, but are not dialable or switched through the PSTN. BRD Broadband - An NXX assigned for multiple Broadband service offerings. Currently, these codes are not ratable in the PSTN; however, this is subject to change. Subscriber bills for usage are generated for Broadband telephone numbers. CDA Customer Directory Assistance only (line number 1212) - This COCTYPE value is used as secondary means of flagging "555" (i.e., DA line numbers only, line number 1212) CO Codes (NXXs). NXX 555 line numbers, beyond x1212 for directory assistance are assigned by NANPA and are not applicable to this document. NXX 555 line number assignment procedures (other than x1212) are addressed in INC document INC94-0429-002. CTV Cable Television - An NXX assigned for subscriber access to a cable television company. The telephone numbers are usually assigned on a 7-digit basis, are ratable on a limited basis, and may be used to determine subscriber charges for selective viewing or special events. ENP Emergency Preparedness - A non-N11 NXX used for emergencies and disaster recovery (i.e. earthquakes, floods, etc.) FGB Feature Group B Access - NXX 950. Used for truck side termination arrangements that provide state FGB originating and terminating exchange access. HVL High Volume - indicates an NXX or block from which the assigned company provides line numbers to address cases involving a high volume of calls over a short period of time (e.g. media promotional call-in requests, certain emergency and relief situations, etc.). The NXX or block involved is not associated with a given Rate Center or switch. Assigned HVL numbers may also often be referred to as “Mass Calling” numbers. Note that when an NXX is dedicated to a specific switch and Rate Center and is involved with “Mass Calling”, it would be considered non-Oddball and represented as either COCTYPE EOC / SSC M (when some lines serve for “Mass Calling”) or COCTYPE EOC / SSC MO (when all assigned lines serve for “Mass Calling”). INP Information Provider - An NXX or block used uniquely for providing various “information services." These include NXX 976 and “976 Like” codes. “976” is reported assigned in an NPA when there is a state tariff (or other regulatory body decision) defining its use. INP records with an SSC of T or W indicate that the entire NXX or block is dedicated to providers provisioning Time or Weather respectively and inclusively. LTC Local Test Code N11 NXXs 211 through 911. Note: - 411, 711, and 911 are nationally designated numbers that should be operational in every NPA in the U.S., U.S. Territories, and Canada - “Activation” of other N11 numbers could vary by state or province, - In Canada, 511 is currently a “reserved code” and 811 is “generally a reserved code” - N11 assignment in other areas of the NANP may vary and are defined by the Code Administrator in each country. Additional N11 information can be found in Section 1.4.2 of the LERG General Section. ONA Open Network Architecture - An NXX assigned exclusively for ONA services. The NXX may be dialable on a company-wide, region-wide, or other limited calling area. RTG Routing Only - An NXX used by an LSP for administrative or official calling purposes. Assigned telephone numbers consist of the pseudo NXX and a four-digit line number. Routing codes are dialable on a 7-digit basis only and are not associated, dialed, translated, nor terminated with a Home or Foreign NPA. Such codes may terminate to multiple locations dependent upon where the call originates, and usually require 7-digit routing and code conversion in the translations process. Routing codes are not rated. UFA Unavailable for Assignment - Certain NXXs may be deemed unavailable for assignment for various reasons. For example, NXXs may be protected in a specific NPA (withheld from assignment) due to certain characteristics and interrelationships between the NPA and NXXs involved, dialing plans, etc. - which, if assigned would create routing and rating conflicts. NXXs may also be considered unavailable for assignment for various other reasons. 700 700 IntraLATA Presubscription - dialing (home NPA)-700-xxxx is used for subscribers’ access into their presubscribed IntraLATA provider’s network. x4141 is reserved to provide an indication to subscribers as to what network they are presubscribed to. NXX 700 can only be assigned as a standard Central Office (CO) Code if the assigned company agrees to reserve x4141 for the indicated purpose. Note: ATCs (Access Tandem Codes) used in Operator Services routing are considered a variation of an ODDBALL Code in that Rate Centers do not apply to them. However, they are not included in the LERG6ODD file since, due to the need to provide the Operator Services Codes associated with them, they are listed separately in the LERG6ATC file. |  |  |
| Contact Information | — | Various LERG sections and files may contain “contact” information fields that provide such information as names, addresses, telephone numbers, etc. The specific fields, lengths, and fieldnames can vary by file. Please note that the contact information, as is true with any such source of this type of information is (1) is not necessarily up-to-date and (2) subjective in its meaning by both the provider and user. TRA data is that supplied by service providers, but over time is subject to become outdated. This is especially true for smaller companies. TRA performs audits and reviews of such data; however, data is subject to becoming outdated before such can even be completed. Also, please note that the provided “contact” is a party that has (or had) some familiarity with the related subject. This does not mean that the contact serves as an across-the-board contact for all functions in a company. |  |  |
| CONTACT FUNCTION, PHONE, INFORMATION | — | This in information maintained directly by the OCN to which it pertains. A specific function is identified as the key to the record, and associated phone and or “other” information is provided. Service of Subpoena exists as the only common function among all companies. (LERG1CON) |  |  |
| COUNTRY | — | The country to which the code is assigned as listed by the ITU. (LERG2, Section 3.5) |  |  |
| COUNTRY CODE | — | The ITU assigned Country Code for a given Country (LERG2, Section 3.5) |  |  |
| COUNTY NAME | — | The name of a County (or equivalent (e.g. Parishes in Louisiana). This is the county with a given state that a 911 Tandem provides service to, in whole or in part. (LERG15) |  |  |
| CREATION DATE IN BIRRDS | — | The date the record was physically created in the underlying database (the Business Integrated Routing and Rating Database System (BIRRDS)). The date should always be a date earlier than the LERG product date. This date is provided for user informational and data reconciliation purposes only. (LERG6, LERG7, LERG7SHA) |  |  |
| CS DATA TDM | — | Circuit Switch Data Tandem. This defines a tandem office which has the functionality (e.g., BCR5, BCR6, and PRI64) to process switched data calls. (LERG7SHA, SOF Indicator) |  |  |
| CSP | __ | Carrier Selection Parameter is a special option that provides for the automatic transmission of a signaling indicator that signifies to the customer whether or not a given call originated from a presubscribed line. (SOF Indicator) |  |  |
| DA OFC (Directory Assistance Office) | — | An office that provides the means for customers or operators to obtain listed telephone numbers and addresses. (Accessed via 411, 555+1212, or NPA+555+1212.) (SOF Indicator) |  |  |
| DA TDM (Directory Assistance Tandem) | — | A tandem office that serves as the concentrated distribution point for customers or operators to access a Directory Assistance Office. (Accessed by dialing 411, 555+1212, or NPA+555+1212.) (SOF Indicator) |  |  |
| DATA DOWNLOAD DATE | — | This is the last possible calendar day through which data entry to the underlying database to the monthly LERG Routing Guide could have been made. (LERGEND) |  |  |
| DATE OF ACTIVITY | — | This is the date (formatted as mmddyy) on which the given activity (defined by Activity Code) was performed in the underlying LERG database. Such dates will generally always be the prior month. However, on occasion, the last day(s) of the month prior to that may appear since (1) the process is based on a LERG-to-LERG timeframe and (2) the LERG is processed, with occasional exceptions, at close of business of the last working day each month (which does not necessarily align to the last calendar day of each month). (Insert files) (See also ACTIVITY CODE) |  |  |
| DERIVED FROM NPA | — | In providing historical NPA information, this indicates the previous NPA(s) that existed in the area covered by a given NPA. In cases of overlays, some/all of the previous NPAs may still cover the area. (LERG3) |  |  |
| DIND | — | This field reflects whether a code is or is not dialable by the customer or operator. Y= yes, dialable, N= no, not dialable. (LERG6, Section 4) |  |  |
| DS | — | Central Office Code (COC, i.e. NXX) STATUS indicator (D, E or M). A blank indicates the record is currently in effect. (See STATUS) |  |  |
| “E” STATUS DATE | — | This is the date that the database underlying the LERG shows as the “establishing” (“E” STATUS”) date of a given record. It could be a past or future date. Although many records may have been established decades ago, the date provided in this field will generally not go back more than 5-7 years. The date is being provided to provide a general indication of how long the given record has been in effect. The field can be used to reconcile near term cases where a record may not have been processed properly on the user end, etc., and should not be used as an historical reference beyond the past several years. AS with any “E” STATUS record, the date signifies the date the Service Provider has indicated the record to be considered established in the network, this is not necessarily the activation date, in-service date, etc., as such terms vary widely in there use. (LERG6, LERG7, LERG7SHA) |  |  |
| EFFDATE | — | The date that a change is to be implemented or effective. The EFFDATE relates to an E, M, or D STATUS code indicating the type of change. With the exceptions of past historical dates in LERG2 and LERG3 and past-month dates in LERG4, all dates are the first day of the month of the LERG or later. Blank values of EFFDATEs indicate the data provided is “current” as of the LERG product date. Date formats are noted in LERGSPEC.DOC and generally are in an mmddyy format. Year dates in LERG3 that are greater than 50 imply 1950+. NOTE: In the case of NPAs 500 and 900 NXX assignments, EFFDATE indicates the planned effective date of the code as indicated to the Numbering/Dialing Planning Organization by the carrier at the time the specific code(s) is allocated to the carrier. NOTE: In Insert files the EFFDATE may, on rare occasions (updates with past dates), be the same as the ACTIVITY DATE. Thus, the EFFDATE reported in these files may not correlate to the past EFFDATE in the underlying LERG database (BIRRDS). This occurs when data is updated using an EFFDATE that is earlier than the then current calendar date. (Insert files) |  |  |
| EMBEDDED OVERLAY NPAs | — | These indicate cases where a certain NPA may be associated (on a grandfathered basis) with a Rate Center. These cases are often excluded from generic NPA maps, etc. and can cause confusion when actual NXX assignments are being assessed. The intent of identifying these NPAs as “Embedded Overlay” NPAs is to distinguish these from those NPAs in a Rate Center from which ongoing NXX assignments are being made. (LERG8) |  |  |
| END OFC (End Office) | — | A switching system that establishes line-to-line, line-to-trunk, and trunk-to-line connections, and provides dial tone to customers. Lineless Hosts are not considered End Offices. (SOF Indicator) |  |  |
| EQPT TYPE | — | A three-character abbreviation for the switching entity's equipment type (LERG7, LERG9, Section 5, Section 7). Examples: Abbreviation Switching Entity Equipment Type 5XB (AT&T-T) No. 5 Crossbar - 2 wire 4E (AT&T-T) No. 4 ESS DMT (Northern Telecom) DMS 10-digital, etc. Equipment type - Switching System Codes are assigned by COMMON LANGUAGE®. A spreadsheet of valid codes is available through www.commonlanguage.com. Click on “Store” in the upper left corner of the web page; then, at the store site, choose code center and then CL-DATA-SSC for the spreadsheet. In the following LATAs, the LERG Routing Guide may not contain switch information and hence the equipment type should not be used. 820 Puerto Rico 822 Virgin Islands 824 Bahamas 826 Jamaica 828 Dominican Republic 830 Other Caribbean Islands |  |  |
| EXPIRATION DATE | — | See TEST LINE EXPIRATION DATE |  |  |
| FG B TDM (Feature Group B Tandem) | — | The tandem switch that serves as the concentrated distribution point for FG B traffic between ICs and end office switches. (LERG7SHA, SOF Indicator). |  |  |
| FG C TDM (Feature Group C Tandem) | — | A tandem office on which end office switches are homed for originating and/or terminating FG C exchange access service. (LERG7SHA, SOF Indicator) FG C provides traditional signaling and is not equipped to provide FG D equal access. |  |  |
| FGD 56 | — | A circuit switched service that provides a trunk side FG D connection from the LEC to the IXC and supports a rate adapted 56Kbps data speed utilizing in-band signaling. (SOF Indicator) |  |  |
| FGD 64 | — | A circuit switched service that provides a trunk side FG D connection from the LEC to the IC and supports 56Kbps or 64Kbps clear channel capability utilizing out-of-band signaling. (SOF Indicator) |  |  |
| FG D TDM (Feature Group D Tandem) | — | A tandem switch that serves as the concentrated distribution point within the LATA or Sector within the LATA as determined by the LEC for FG D traffic between switching entities and the Interexchange Carriers. Allows a caller to access a presubscribed IC by dialing 1+10-digit telephone number, and any other IC by dialing the corresponding 101XXXX access code plus the telephone number. Provides trunk side equal access with “1+”/“101XXXX”dialing. Note that in LERG14, the identified switch (record key) is a FG D or OS Tandem. (LERG7SHA, LERG14, SOF Indicator). |  |  |
| FGD ADJ EO (Feature Group D Adjunct End Office) | — | An end office that has an external device (adjunct) that provides equal access service to the subscribers in that office. (SOF Indicator) |  |  |
| FUNCTION | — | See TDM FUNC CODES |  |  |
| FOOTNOTE | — | Where this appears this is providing some additional information about the given data element. It exists to permit flexibility for the data provider to describe unique characteristics of the data. |  |  |
| FOOTNOTE CODE | — | In the Country Code data, this footnote code should be looked up in the Country Code section of the Glossary. (LERG2) |  |  |
| FGD ADJ TDM (FG D Adjunct Tandem) | — | A toll tandem switch equipped with the equipment and software features that allow non-conforming end office to switch traffic to an Interexchange Network as equal access. (SOF Indicator) |  |  |
| GENERAL | — | See CATEGORY. |  |  |
| HC (Horizontal Coordinate) | — | A four or five digit number used, in association with the VC that relates to the geographical location of a Switching Entity / POI. These coordinates are derived from Latitude and Longitude values. The VC and HC are used to measure the airline mileage between like entities (i.e., Switching Entity to Switching Entity). IMPORTANT: Do not confuse Vertical and Horizontal Coordinates of a switch with those of a Rate Center (See MAJOR RC VC / HC) – they may sometimes be identical, sometimes may not. (Switch V&H: LERG7, Section 5; RC V&H: LERG8, Section 6) |  |  |
| HOST | — | A switching office that provides certain common processor functions for a remote entity and for the traffic that originates and/or terminates in the remote. (LERG7SHA, SOF Indicator) |  |  |
| HS | — | Homing STATUS indicator (See D, E, M). A blank indicates the record is currently in effect. (LERG9, Section 7) |  |  |
| IAC (Interexchange Access Customer Code) | — | A four character alphabetic code in the OCN field and is used to identify certain interexchange carriers (primarily NPA 500 and 900). The alpha codes are proprietary, COMMON LANGUAGEâ codes assigned by the Telcordia Technologies Language Standards Department. |  |  |
| IC | — | Interexchange Carrier - a carrier that is authorized by the Federal Communications Commission (FCC) to carry wireline based traffic between Local Access Transport Areas (LATAs). Often referred to a long distance carrier or IC. (See CATEGORY) |  |  |
| ICO | — | Independent Telephone company - the initial telephone company that provides wireline local exchange service in a non-RBOC geographical area. This includes Southern New England Telephone and Cincinnati Bell, Inc. ICOs and RBOCs are often referred to as the “incumbent” local exchange carrier. (See CATEGORY) |  |  |
| IDDD (International Direct Distance Dialing) | — | A single-alpha value (Y/N) used to indicate whether or not the switching entity has IDDD capability. (LERG7, Section5) NOTE: Y indicates switch provides direct dialing for international calling. Signaling will be dependent upon the end office capability. Non-equal access offices will utilize two-stage outpulsing. Equal access offices will utilize Feature Group D signaling. |  |  |
| Insert files | — | In the data versions of the LERG, “Insert” files exist for LERG6, LERG7, LERG7SHA, and LERG12. Insert files provide a means to identify data record changes on a month-to-month basis. Since a given LERG is based on a monthly snapshot of a dynamically changing database, records appearing in an Insert file may have an EFFDATE in the prior month, or, a date in the future. For example, database activities performed after a March LERG is created (at beginning of month) may yet have an EFFDATE in March. This change would then appear in the appropriate “insert” file provided with the June LERG. Similar database activity in March may have an EFFDATE in a future month such as October as well. Note that Insert files are NOT comprehensive lists of ALL future activity reported in the main file. Since data will be reported in the LERG with an EFFDATE until that date is past, the given EFFDATE for the key involved will appear, perhaps only once, in an Insert file (e.g. following the month of its addition). (See ACTIVITY CODE, DATE OF ACTIVITY and VERSION). Need and use of Insert files is at the user’s discretion, based on how the primary files are being used, the extent of any need to reconcile near term month-to-month data changes, etc. These files essentially provide a means to reconcile and address changes to current data in the current LERG (new records, lost records, changed records) for which no indication of such occurring was provided in the previous LERG and to isolate future activity not previously reported. |  |  |
| INTERM OFC (Intermediate Office) | — | Identifies a switching entity that performs an “intermediate” office/tandem function in certain network architecture arrangements. An “intermediate office” is a switching entity, other than the originating or terminating end office or access tandem, used to assist in the call completion process. (SOF Indicator) |  |  |
| INTRA PRSUB | — | A switching entity that has Dual PIC capability. This allows a subscriber IntraLATA equal access. This gives them the ability to pre-subscribe an IntraLATA Carrier that may be different from their current InterLATA Carrier. (SOF Indicator) |  |  |
| INTRA TDM | — | An IntraLATA tandem switch connects one trunk to another and serves as a trunk concentration and distribution function to minimize direct end office interconnection. IntraLATA tandem traffic can be either intrastate IntraLATA or interstate IntraLATA as defined in the tariffs on file with the appropriate regulatory body. An IntraLATA tandem completes billable toll messages that originate and terminate within the same LATA. A switch that completes IntraLATA toll traffic between subtending end offices. (LERG7SHA, SOF Indicator) |  |  |
| INTRALATA | — | IntraLATA Toll provider - a company that exists to provide an alternative to IntraLATA toll services that is provided by local service providers. (See CATEGORY) |  |  |
| ISDN FS OFC | — | ISDN Foreign Serving Office. A single 11-character field that indicates, when appropriate, that ISDN services are available to subscribers in a given office; however, they are actually “served” by the identified “foreign” (i.e., different) office. (SOF Indicator) |  |  |
| ISDN MultiRate | — | A circuit switched service that allows customers to set up n x 64Kbps (n by 64) calls from an ISDN Primary Rate Interface circuit in real time and in the same manner as any circuit switched ISDN call. ISDN MultiRate is an extension of the 64Kbps service offering in that it can set up a call from 64Kbps to 1,536Kbps (1 DS0 to 24 DS0s) in bandwidth capacity. (SOF Indicator) |  |  |
| L RESELLER | — | Local Reseller - A company that leases a block of numbers or facilities, in bulk, from a local exchange carrier, for purposes of resale to customers. (See CATEGORY). |  |  |
| LARGE NETWORK | — | Used for Signaling System 7 (SS7) Network Code assignments. More than 75 initial signaling points, or 150 projected signaling points within five years. The network must have Signaling Transport Point (STP) or STP functionality. |  |  |
| LAST KNOWN ADDRESS | — | As with any database containing contact information, such information is difficult to keep up-to-date for numerous reasons. This field, when populated with an “X”, indicates that the address information provided has been reported as invalid to TRA and confirmed to be no longer valid. However, rather than removing the information, it has been retained in case it can be useful for some reason. Telcordia TRA does attempt to resolve such cases, however, situations such as a defunct company, whose OCN information is needed to be retained for some reason, would be a case where such information cannot be readily updated. Should you become aware of any information that can assist in resolving LKA situations, please call the TRA Customer Service Center on 866-NPA NXXS (866-672-6997) or (732-699-6700). (LERG1) |  |  |
| LAST MODIFICATION DATE | — | For currently active records, this is the latest date (prior to the LERG production date) that, based on current information in the underlying database to the LERG, activity (changes) to the record was last effective. It is possible, if no changes have since occurred, that this date is the same as the “E” STATUS DATE. For future establishing records, this is the establishing date of the record. (LERG6, LERG7, LERG7SHA) |  |  |
| LATA | — | This is the three-digit number that represents the geographical Local Access and Transport Area (LATA), or LATA-like code, in which this particular record is located, e.g., 120 for Maine. Note that some descriptions indicate five positions for the LATA code. The last two digits are for the LATA sub-zone (in Florida only) which represent Equal Access Exchange Areas (EAEAs). (Various files and sections) Notes: - In some cases, for instance where the switch serves NXX’s in two LATAs, the LATA entry might be different from the geographical LATA of the switch. This entry identifies the LATA and, therefore, the Point of Interface (POI) to which Interexchange Carriers deliver traffic for access to the NXX’s assigned to the particular switch. (Various files and sections) - It is possible, depending on circumstances, for a switch to be in one LATA and its tandems in another; that an NXX may be associated with a Rate Center that is in one LATA, but is served by a switch in another LATA. Although such LATA interrelationships, in the majority of cases, are all coincident, it is nonetheless important to ensure you are clear as to what data you are referring to when attempting to associate LATAs to it. This is especially critical when considered an NPA NXX BLOCK record (i.e. is the LATA you require that of the Rate Center, Switch, Tandem, etc.). - LATA 99999 may be found for some records. These are generally cases of ODDBALL NXX’s or associated data where the data is not necessarily associated with a single LATA. - LATAs were created as part of the 1984 AT&T Divestiture process and based largely on the then existing network. Some LATAs were defined during the year or two after divestiture. The geographical boundaries of LATA generally are within state boundaries, but do occasionally cross state boundaries. - LATAs technically only pertain to FCC managed areas in the NANPA. For purposes of data consistency, however, LATAs are associated with other NANPA countries (e.g. 888 is the entire country of Canada). |  |  |
| LATA NAME | — | The name of the Local Access and Transport Area (LATA), or LATA-like code, e.g., Maine. (various files and sections) |  |  |
| LERG | — | The Telcordia® LERG™ Routing Guide |  |  |
| LERG FILE NUMBER | — | An eight-character value that signifies the specific LERG Routing Guide file that associated information in LERGEND is provided for. (LERGEND) |  |  |
| LINES FROM/TO | — | Two four-digit elements representing numbers served by the associated switching entity for the NPA NXX (i.e., both working and spare). The first four digits represent the starting number in the block of numbers. The last four digits represent the last number in the block of numbers. In LERG6, for “A” BLOCK IDs, the FROM will be 0000 and TO will be 9999 with the exception of NXX 555 which will have 1212 and 1212 respectively; numeric blocks 0-9 should be 0000-0999, 1000-1999, etc. where the first digit of the FROM and TO is the same as the BLOCK ID. Note that a “full set” of BLOCK IDs (0-9) should not be assumed; therefore, a full complement of ranges equating to 0000-9999 should not be assumed for NXX records that have numeric BLOCK IDs. (See TBP IND, BLOCK ID) (LERG6, Section 4) |  |  |
| LNP CAPABLE | — | LNP (Local Number Portability) Capable signifies the switch is able to process SS7 LNP messages. (SOF Indicator) |  |  |
| LOCAL TDM | — | A Local Exchange Carrier (LEC) switching system, specifically identified as a Local Tandem in the LERG Routing Guide, which provides a traffic concentration and distribution function for local traffic originating and/or terminating within a local calling area as defined in the state tariff(s) on file with the appropriate regulatory body. A Local Tandem provides trunk-to-trunk connections to more than one end office within a local calling area. Although interconnection at more than one Local Tandem may be required to provide access to all end offices within a local calling area, only the “homing” or “subtending” interconnection is reflected in the LERG Routing Guide. Also, there may be end offices that do not subtend nor interconnect with a Local Tandem. A host/remote scenario does not constitute a Local Tandem homing arrangement. Nor should an office be considered a Local Tandem if local traffic is routed to that office solely for emergency or special routing arrangements, e.g., Type 2A Interconnected wireless. (LERG7SHA, SOF Indicator) |  |  |
| LOCALITY | — | This is the name of the locality served by a COC. The locality entered often is what appears as the called place on a customer's bill. (LERG6, LERG8LOC, LERG11, Sections 4,6,9) NOTE: The Locality Field may consist of ten characters. In those cases where the Locality name exceeds ten-characters it has been abbreviated. LERG8LOC has a mapping of the 10-character Locality name to a spelled out max of 50 characters. |  |  |
| LOCATION (NPA) | — | This is the NANP assigned name of the NPA, e.g., Maryland (301) (LERG3, Sections 3.1, 3.2). |  |  |
| LRN | — | Location Routing Number used in support of Local Number Portability. (LERG12, Section 10) |  |  |
| LRN Type | — | LRN Type field provides the capability to distinguish between LRN types. ("P" = primary, "M" = maintenance). Actual use of these fields may vary by company. This is not a standardized field in LNP processes. (LERG12, Section 10) |  |  |
| LS | — | Location Routing Number (LRN) STATUS indicator. (LERG12, Section 10) |  |  |
| MAJOR RC–VC | — | Major Rate Center Vertical Coordinates. A five-digit number used with the RC-HC (Rate Center Horizontal Coordinates) to pinpoint the location of a Rate Center. The V&H Coordinates of two Rate Centers can be used to calculate the airline mileage between them. Major V&H Coordinates are used to rate Message Telephone Service (MTS) calls when the calling and called NPA/NXX’s are in Rate Centers that are farther apart than a number of airlines specified in the billing company's tariff. (See also MINOR RC-VC.) Note that if a company does not specify in their tariff the conditions for using major and minor Rate Center V&H coordinates, the major coordinates are used in all cases. (LERG8, Section 6) IMPORTANT: Do not confuse Vertical and Horizontal Coordinates of a switch with those of a Rate Center (See VC / HC) – they may sometimes match, sometimes may not. (Switch V&H: LERG7, Section 5; RC V&H: LERG8, Section 6) |  |  |
| MAJOR RC–HC | — | Major Rate Center Horizontal Coordinates. See MAJOR RC-VC (Major Rate Center Vertical Coordinates) (LERG8, Section 6) |  |  |
| MEMBER | — | Generally viewed as the last of the three sets of three numbers comprising a point code (NETWORK-CLUSTER-MEMBER). (LERG4) Note: Assignments made by for small companies are done in sets of 4 four numbers defined by member-from and member-to. |  |  |
| MINOR RC–VC | — | Minor Rate Center Vertical Coordinates. A five-digit number used with the RC-HC (Rate Center Horizontal Coordinates) to pinpoint a Rate Center. The V&H Coordinates of two Rate Centers can be used to calculate the airline mileage between the two Rate Centers. Minor V&H Coordinates are used to rate Message Telephone Service (MTS) calls when the calling and called NPA/NXX’s are in Rate Centers that are closer together than a number of airline miles specified in the rating company's tariff. Note: Minor V&H Coordinates in Alaska are computed using a different algorithm that major V&H coordinates. (See also MAJOR RC-VC.) (LERG8, Section 6) |  |  |
| MINOR RC–HC | — | Minor Rate Center Horizontal Coordinates. Note: Minor V&H Coordinates in Alaska are computed using a different algorithm that major V&H coordinates. (See also MINOR RC-VC) (LERG8, Section 6) |  |  |
| MTA1, MTA2 | — | Major Trading Area (MTA) is a coded value (see table below) used by the U.S. Federal government for determining service areas for some wireless service providers. Data in the LERG maps a Rate Center to the MTA in which it exist. Due to noncontiguous boundaries between MTAs and Rate Centers there may, on occasion, be two MTAs associated with a Rate Center. (LERG8) 01 New York 02 Los Angeles-San Diego 03 Chicago 04 San Francisco-Oakland-San Jose 05 Detroit 06 Charlotte-Greensboro-Greenville-Raleigh 07 Dallas-Fort Worth 08 Boston-Providence 09 Philadelphia 10 Washington-Baltimore 11 Atlanta 12 Minneapolis-St. Paul 13 Tampa-St. Petersburg-Orlando 14 Houston 15 Miami-Fort Lauderdale 16 Cleveland 17 New Orleans-Baton Rouge 18 Cincinnati-Dayton 19 St. Louis 20 Milwaukee 21 Pittsburgh 22 Denver 23 Richmond-Norfolk 24 Seattle (Excluding Alaska) 25 Puerto Rico-U.S. Virgin Islands 26 Louisville-Lexington-Evansville 27 Phoenix 28 Memphis-Jackson 29 Birmingham 30 Portland 31 Indianapolis 32 Des Moines-Quad Cities 33 San Antonio 34 Kansas City 35 Buffalo-Rochester 36 Salt Lake City 37 Jacksonville 38 Columbus 39 El Paso-Albuquerque 40 Little Rock 41 Oklahoma City 42 Spokane-Billings 43 Nashville 44 Knoxville 45 Omaha 46 Wichita 47 Honolulu 48 Tulsa 49 Alaska 50 Guam-Northern Mariana Islands 51 American Samoa |  |  |
| NANP | — | The North American Numbering Plan. This covers a group of countries (United States and some of its territories, Canada, and certain islands in the Atlantic and Caribbean) that follow a common numbering plan in the generic form of 1-NPA-NXX-xxxx. The NANP has also been referred to as World Zone 1 within the World Zones defined by the International Telecommunications Union (ITU). |  |  |
| NANPA | — | North American Numbering Plan Administration/Administrator. NANPA administers assignments of certain codes (e.g. CIC Codes, 555 Line Number Assignments, etc.) for the countries that constitute the North American Numbering Plan. NANPA also serves as a centralized CO Code Administrator (assignments of NXX’s) to companies serving areas of the United States and United States Territories that participate in the NANP. |  |  |
| NETWORK | — | Generally viewed as the first of the three sets of three numbers comprising a point code. (NETWORK-CLUSTER-MEMBER) (LERG4) |  |  |
| NPA | — | The Numbering Plan Area code is the first three digits of the Destination Code. The NPA, combined with the Central Office Code (COC), is the Destination Code being reported. (various files and sections) |  |  |
| NXX | — | In the LERG Routing Guide, this may also be referred to as a Central Office Code (COC) or as a Destination Code. NXX’s are technically the three digits following the NPA (Area Code) in the numbering schema used by countries participating in the North American Numbering Plan (NANP). |  |  |
| OCN (Operating Company Number) | — | This four-position alphanumeric field is a method for identifying an NPA-NXX code-holder, switching entity company, non-facility-based service providers such as resellers, billing service providers, etc. The term has been defined by TRA and employed in this capacity since 1984. A complete listing of OCNs and the “names” of the companies they refer to is contained in the LERG. (Sections 2.1, 2.2, LERG1) (Note: OCN appears in various files and sections) TRA assigns OCNs by utilizing the NECA (National Exchange Carrier Association)-assigned Company Codes when such exist. If a company does not require a Company Code to be assigned, yet needs an OCN for various other reasons, the Telcordia® Routing Administration (TRA) may uniquely assign an OCN to that company. TRA assignment of OCNs includes, but is not limited to, the following types of situations: - OCNs of NXX’s within Service Access Codes. - Administrative OCNs (e.g. AOCN-only companies). - OCNs associated with Access Tandem Codes. |  |  |
|  |  | Procedures are in place to ensure that: - Any company (facility-based or non-facility-based) that needs an OCN should obtain a NECA Company Code as outlined in the NECA Company Code Assignment Guidelines. NECA assignments (new, as well as changes to existing) are forwarded to TRA by NECA. TRA will then use these assignments as the previously mentioned subset of its OCN values. (Contact NECA at 973 884-8355 regarding Company Codes). - Companies that have been assigned NECA Company Code(s) may use one or more of such codes to represent their data in the LERG. - OCN lists in TRA databases will include all NECA Company Code assignments, and will include TRA assigned OCNs as noted earlier. Note to avoid confusion with NECA Company Codes that follow an nXXX format (n = numeric, X = alphanumeric), TRA assigned OCNs follow an aXXX format (a = alpha, X = alphanumeric). The intent of this definition includes: - To provide for the identification of service providers that are non-facility-based (such as billing service providers) and facility-based. - To permit TRA identification of companies which have only administrative responsibilities within TRA databases but which have no other need for OCNs or NECA-assigned Company Codes. - To ensure that there is an exact, 1-to-1 relationship between NECA assigned Company Codes that are used as OCNs (i.e., that all NECA Company Codes constitute a subset of OCNs, and that information and codes pertain to the identical companies). For database management needs, some OCNs may not correspond to an assigned Company Code: Gxxx = General - primarily used to identify an AOCN (Administrative Operating Company Number) when the administrative company itself does not provide telecommunications services to subscribers, or cases where the company has assigned Company Codes, but opts to have TRA assign a unique code for use as its AOCN value. Gxxx is also used to identify non-OCN or non-AOCN companies requiring query-only access to the underlying database. Ixxx = Interexchange Carrier, where xxx is an ACNA (Access Customer Name Abbreviation) code - used for IC (Interchange Carrier) assignments of NPA 500 and 900 NXX’s and occasionally for operator tandem interfaces. |  |  |
|  |  | Pnnn = PCS (Personal Communication Services) assignments (NPA 500) made to companies that do not have an OCN or can otherwise be classified as an Ixxx OCN. RECx = Represents NPA NXX BLOCKs that are in the process of being “reclaimed” by the CO Code or Pooling Administrator. TRAx = Telcordia Routing Administration Other OCNs may be formatted as: Cxxx (not LERG related), MULT (data applies to multiple companies), NONE (not LERG related), REC1, SNxx (not LERG related), TCAN, UNAS (not LERG related). Notes: - As with certain other data elements provided in the LERG, when using OCN values in reports, cross relating with other information, etc., please ensure you understand the extent and limitations of the data. For example, the company assigned a given NXX may operate that NXX through a switch belonging to another company that, in turn, may have another company as a tandem provider. Therefore, it is critical to understand the nature of data requests, the nature of data provided on other reports, etc., to ensure proper use of the LERG data. (See OVERALL OCN) - The OCN associated with an LRN (LERG12) is that of the NPA NXX. This may be different from that of the switch owner (LERG7) or what may have been provided by the service provider to NPAC as the SPID associated to the LRN. |  |  |
| OCN NAME | — | This is a twenty-character (for abbreviated Operating Company Name value) or a fifty-character (for the full Operating Company Name value) field, e.g., ACKERMAN TEL CO. (LERG1, Sections 2.1,2.2) |  |  |
| ODDBALL Codes | — | See listing and descriptions under COCTYPE. (LERG6, LERG6ODD) |  |  |
| OFF SHORE | — | Those locations outside the continental portion of the U.S. that form part of the NANP and are supported by BIRRDS, e.g., Alaska, Hawaii, some Caribbean Islands. |  |  |
| OPR SVC CODE | — | An operator-only dialable code assigned for a specific service when used by an operator services subsystem to access operator services outside the originating operators’ system. A listing of these codes can be found in Section 1.4, Routing Codes. (LERG6ATC, LERG9ATC, LERG11, LERG12, Sections 4, 7, 8, 9) |  |  |
| OS TDM (Operator Services Tandem) | — | An Operator Services (OS) tandem switch serves as the concentrated distribution point for providing a host of services that may include toll and intercept. The OS Tandem is an integral part of the network as it performs Alternate Billing Services, Automated Coin Telephone Service, Automatic Message Accounting (AMA) teleprocessing, and Automatic Call Distribution for operator handling of calls. Note that in LERG14, the identified switch (record key) is a FG D or OS Tandem. (LERG7SHA, SOF Indicator) |  |  |
| OVERALL OCN | — | This field identifies the “Overall” Company Code (referred to by TRA as an “Overall” OCN) as assigned by the National Exchange Carriers Association (NECA) companies. Not all OCNs are Company Codes, and not all Company Codes have “Overall” Company Codes; therefore, cases will exist where the OVERALL OCN field is blank. “Overall” OCNs can be used when attempting to identify OCNs that all may be related to the same “company.” However, bear in mind that the term “company” itself is imprecise. Totally separate “companies” may have the same “name,” but be operating in different states. Likewise “company” XYZ and XYZ Wireless may corporately be under the same umbrella, but for various reasons, at various times, may be treated as separate “companies.” (LERG1) |  |  |
| P RESELLER | — | PCS Reseller. (See CATEGORY) |  |  |
| PACKET E.164 | — | Indicates the switch provides IntraLATA X.25 packet ISDN packet data access. Customers are assigned E.164 addresses. (SOF Indicator) |  |  |
| PACKET X.121 | — | Indicates the switch provides IntraLATA X.25 packet data access. Customers are assigned X.121 addresses. (SOF Indicator) |  |  |
| PAIRED CODE | — | Paired Country Code (a.k.a. pseudo-Country Code) (LERG2, Country Code list): a paired Country Code is an AT&T assigned Country Code for non-equal access originated ILDS (International Long Distance Service) calls. These codes provide a 3-digit code for translation. The 3-digits are either the actual 3-digit ITU assigned Country Code, the actual 1-2 digit ITU assigned Country Code filled with leading “0” to ensure a 3-digit code, or 3 totally unrelated digits, e.g., the ITU assigned Country Code for Russia is “7”, and the paired Country Code is “007”. (LERG2, Section 3.5) |  |  |
| PC (flag) | — | Point Code - a nine-digit numeric element used to identify a particular node in the Signaling System 7 (SS7) network. (LERG7, Section 5) To indicate the existence, non-existence or change in the Point Code the following means of “flagging” will be used: - If the SWITCH currently, or at a future effective date, does not have a Point Code (or has one “removed”), the current (or future) view of the SWITCH record will contain a blank in the Point Code field. - If the SWITCH record currently has a Point Code, or will have one placed on it in the future, the “flag” will contain a “P”. A “P” will exist on all future views of this record as well, unless the cases above or below apply. - If a sequential future view of the SWITCH record finds that the previous view (non-blank) value of a Point Code is "changed" to another (non-blank) value, the flag will contain a “C”. Unless the view after the “C” has also changed value, the value of the “flag” will revert to a “P”. |  |  |
| PCS | — | Personal Communications Service - a company that exists to provide Personal Communications Service to its customers pursuant to FCC Regulations Part 24. (See CATEGORY) |  |  |
| PERMISSIVE DIALING | — | This is a term used with NPA (Area Code) Splits. Permissive dialing is a period when two dialing situations (dialing the name NXX in two different NPAs; seven/ten digit dialing; etc.) occur simultaneously to arrive at the same terminating point. For purposes of the LERG Routing Guide data, permissive dialing is the period when a subscriber may dial someone using either that person’s new or old Area Code (NPA). |  |  |
| PMC | — | Public Mobile Carrier Type II Interconnected at the tandem. See COCTYPE. (LERG6, LERG9, Section 4) |  |  |
| PORTABLE | — | A ‘Y’ in this field indicates that at least one line number in the NPA NXX BLOCK may be ported either due to Thousands-Block-Number Pooling and/or Service Provider Local Number Portability. Porting involves mapping a given line number to a Location Routing Number (LRN) via the Number Portability Administration Center (NPAC) for routing the call (i.e., the basic a process involved with Local Number Portability, (LNP). (LERG6, Section 4) Note: Line numbers associated with paging are not to be considered Service Provider portable. In cases were an entire NPA NXX or an entire thousands block is dedicated to paging numbers, the PORTABLE indicator will be “N”. |  |  |
| POSTAL CODE | — | See ZIP CODE |  |  |
| PRI 64 | — | An Integrated Service Digital Network (ISDN) Primary Rate Interface (PRI) access capability that allows a customer premise device to communicate directly with the network and/or another ISDN equipped location, utilizing an out-of-band protocol and has data rates of 56Kbps, 64Kbps clear, or multiple combinations of 56 or 64Kbps clear. PRI is 23 64Kbps clear channels, which can be used for any combination of voice and data, and one 64Kbps data channel that is used for signaling (23B+D). (SOF Indicator) |  |  |
| RATE CENTER | — | A Rate Center is technically the approximate midpoint of what is usually called a Rate Exchange Area, although the term Rate Center has also been used synonymously with the geographic area itself. A Rate Center is a point within a uniquely defined Rate Exchange Area from which mileage measurements are determined. Rate Exchange Area and Rate Center information, as well as other aspects (e.g. V&H) are addressed and defined in local exchange tariffs filed with each state commission by local service providers operating in each state. Specifics of which companies file such tariffs, extent of the content, formats, etc., although consistent in several aspects across different states, may vary by state. Local Calling Area is the basic local area defined for calls within a Rate Center (usually all local) as well as calls from a given Rate Center to other Rate Centers (may be local or toll). This information is NOT in the LERG Routing Guide. Local Calling Area and specifics about expanded/extended Local Calling Areas are defined in state tariffs. |  |  |
| RATE CENTER | — | LERG6 files (Section 4) “map” NPA NXX’s (and BLOCK IDs) to Rate Centers. LERG8 files (Section 6) provides information such as Rate Center V&H, Locality names, etc. NOTE: The RC TYPE field is used to identify Rate Centers requiring special identification. The following are examples of RC TYPEs identifying a particular Rate Center: Unrestricted Rate Center provides a range of Tele- (b = blank) communications Services and is not restricted to a specific function. Suburban Unit established to further define large exchange Zone areas. Suburban Zones apply to large metropolitan - areas and may include only the area around a city (e.g., Pittsburgh Suburban Zones) or the city and its surrounding area (e.g., Wheeling Suburban Zones). The exchange area must be large enough to warrant a subdivision of two or more suburban zones. Suburban Zones are assigned a vertical and horizontal coordinate for use in measurements between Rate Centers, suburban zones or Zoned Cities, in the same manner as Rate Center vertical and horizontal coordinates. Zoned City Unit established to further define large exchange (Z) area usually encompassing a city (e.g., New York City). Each zoned city will be assigned a vertical and horizontal coordinate (identified as the “Major Zone”). In addition, the zoned city will be sub-divided into two or more city zones. Vertical and Horizontal coordinates will be assigned to each city zone to be used in the same manner as suburban zone vertical and horizontal coordinates. |  |  |
| RAO | — | A Revenue Accounting Office (RAO) is a three-character code that may contain alphas and/or numerics. This is a code that is used in various billing systems for purposes of collating various billing records with in those processes to ensure permit calls and related telephony charges to be properly billed by and to the proper parties. Bill-to RAOs are high-level collection nodes within this concept. When further breakdowns are needed, Send-to RAOs are used. A Send-to RAO should always have an associated Bill-to RAO. A Bill-to RAO may have none, or several, associated Send-to RAOs. The RAO Administrator in Telcordia Technologies, Inc assigns RAOs. To be directed to the RAO administrator, you may call the TRA Customer Service Center. |  |  |
| RBOC | — | Regional Bell Operating Company - refers to, on a high level, to one of seven corporations created to provide local exchange service as part of AT&T's 1984 divestiture (Ameritech, Bell Atlantic, BellSouth, NYNEX, Pacific Bell, Southwestern Bell, US WEST). On a lower level, this can refer to any one of the 22 Bell Operating Companies (BOCs) that, in 1984, were incorporated into one of the seven RBOCs. See CATEGORY. Subsidiaries of the RBOCs and/or BOCs created to support wireless services, competitive local exchange service (i.e., outside their traditional areas), etc., are not included in this definition. These would fall under the appropriate category (e.g. CLEC), as may apply. |  |  |
| RC | — | Rate Center. (See RATE CENTER) |  |  |
| RC TYPE | — | Rate Center Type. (See RATE CENTER) |  |  |
| RCC | — | Radio Common Carrier-Dedicated Type 1 Interconnected at the end office. See COCTYPE. (LERG6, Section 4) |  |  |
| RECORD COUNT | — | Count of records in each data file for a given monthly LERG Routing Guide. (LERGEND) |  |  |
| RECORD TYPE | — | Indicates if a given “line” of data pertains to the tandem itself (Record Type = A); ATC codes (if any) using that tandem (B); to subtending NPA/NXX and switch information (C). (LERG9) |  |  |
| RELEASE DATE | — | This is the “product date” of the monthly LERG Routing Guide (i.e., the first of each calendar month). (LERGEND). |  |  |
| REMOTE | — | A switching office that is dependent on another office (the “host”) for certain common processor functions; usually originating and terminating traffic for the remote is provided via the host switch. (SOF Indicator, Section 5, LERG7SHA, SWITCH.) |  |  |
| RS | — | Rate Center status indicator (See D, E, or M). A blank indicates the record is currently in effect. (See STATUS) |  |  |
| SEND-TO RAO | — | See RAO. (LERG7) |  |  |
| Serving Wire Center | — | The LERG does not contain a specific data element entitled “Serving Wire Center”. Use of this term is somewhat ambiguous within the telecommunications industry. It may, depending on definition being used, refer to data that is in the LERG such as SWITCH, or the first 8 characters of the SWITCH identifier. Serving Wire Centers also may be referred to sometimes by “names” which are not identified as such in the LERG. |  |  |
| SHA IND | — | Switch Homing Arrangement (SHA) Indicator identifies the “homing” arrangement to be used for the NPA/NXX, relative to the entered 11-character switch. If populated with a value between 01 and 99 the combination of the 11 character switch identifier and the 2 digit SHA indicator represent an alternative homing arrangement that must be previously established as an SH2 record within BIRRDS. In correlating the LERG6 and LERG7SHA files the SHA value must be used in conjunction with the SWITCH to obtain the appropriate homing for a given NPA NXX. (LERG6, LERG7SHA, LERG9, LERG12) |  |  |
| SMALL NETWORK | — | When used with Signaling System 7 (SS7) Network Code assignments. Fewer than 75 signaling points. The network must have Signaling Transport Point (STP) or STP functionality. |  |  |
| SPLIT INDICATOR | — | This indicator will be set to ‘Y’ when the associated Rate Center is geographically split across NPA boundaries. Please not that this indicator does not pertain to NPA (Area Code) splits. (LERG8) |  |  |
| SOF | — | Switching Entity - Office Functionality This list identifies the functionality that a switching entity has been designated to perform. SOF Indicators are listed in the order they appear in LERG Routing Guide data products. Note that this is intended to identify what functions are (or will be) performed, not necessarily what functions “could be” performed if a specific release, process, etc., were implemented. In the LERG Routing Guide data files, the applicability of a given functionality is noted with an “X” in the field. (LERG7, Section 5) |  |  |
|  |  | SOF | FUNCTIONALITY TERMS | DEFINITIONS |
|  |  | 1 | END OFC | End Office |
|  |  | 2 | HOST | Host |
|  |  | 3 | REMOTE | Remote |
|  |  | 4 | DA OFC | Directory Assistance Office |
|  |  | 5 | CLASS 4/5 | Class 4 And Class 5 Function |
|  |  | 6 | WIRELESS OFC | Wireless Office |
|  |  | 7 | FG D ADJ EO | Feature Group D Adjunct End Office |
|  |  | 8-11 | filler |  |
|  |  | 12 | FG B TDM | Feature Group B Tandem Function |
|  |  | 13 | FG C TDM | Feature Group C Tandem Function |
|  |  | 14 | FG D TDM | Feature Group D Tandem Function |
|  |  | 15 | OS TDM | Operator Services Tandem |
|  |  | 16 | INTERM OFC | Intermediate Office |
|  |  | 17 | DA TDM | Directory Assistance Tandem |
|  |  | 18 | 911 TDM | 911 Tandem |
|  |  | 19 | FG D ADJ TDM | Feature Group D Adjunct Tandem |
|  |  | 20 | LOCAL TDM | Local Tandem |
|  |  | 21 | INTRA TDM | IntraLATA Tandem |
|  |  | 22 | CS DATA TDM | Circuit Switched Data Tandem |
|  |  | 23 | BCR5 | Basic Rate Interface Data Rate Of 56kbps |
|  |  | 24 | BCR6 | Basic Rate Interface Data Rate Of 64kbps |
|  |  | 25 | PRI 64 | Primary Rate Interface |
|  |  | 26 | ISDN MULTIRT | Integrated Service Digital Network Multirate |
|  |  | 27 | ISDN FS OFC | Integrated Service Digital Network Foreign Serving Office |
|  |  | 28 | X.75 GATEWAY |  |
|  |  | 29 | PACKET X.121 |  |
|  |  | 30 | PACKET E.164 |  |
|  |  | 31-33 | filler |  |
|  |  | 34 | STP | Signaling Transfer Point |
|  |  | 35 | CCS AC OFC | Common Channel Signaling Access Capability Office |
|  |  | 36 | filler |  |
|  |  | 37 | 800 SSP | Switch Can Perform An 800 Database Query |
|  |  | 38 | LNP CAPABLE | Local Number Portability Capable |
|  |  | 39 | filler |  |
|  |  | 40 | filler |  |
|  |  | 41 | filler |  |
|  |  | 42 | CIP | Carrier Identification Parameter (SS7) |
|  |  | 43 | CSP | Carrier Selection Parameter (SS7) |
|  |  | 44 | filler |  |
|  |  | 45 | SW56 | Switched 56kbps Service |
|  |  | 46 | FGD 56 | Feature Group D 56kbps Service |
|  |  | 47 | FGD 64 | Feature Group D 64kpbs Service |
|  |  | 48 | INTRA PRESUB | IntraLATA Presubscribed Service |
|  |  | 49 | CALL AGENT | Switch is a Call Agent |
|  |  | 50 | TRUNK GATEWAY | Switch is a Trunk Gateway |
|  |  | 51 | ACCESS GATEWAY | Switch is an Access Gateway |
| SP1 | — | Service Provider Type 1. COC TYPE data entry used to identify a dedicated NPA NXX BLOCK assigned to a Service Provider (Type 1 interconnected) offering miscellaneous types of service. Utilizing an NPA NXX BLOCK, the service provider may offer a variety of services to end user subscribers, e.g., ISDN services, non-500 Personal Communications Services (PCS), Voice Mail, etc. (See COC TYPE) |  |  |
| SP2 | — | Service Provider Type 2. COC TYPE data entry used to identify a dedicated NPA NXX BLOCK assigned to a Service Provider (Type 2 interconnected) offering miscellaneous types of service. Utilizing an NPA NXX BLOCK, the service provider may offer a variety of services to end user subscribers, e.g., ISDN services, non-500 Personal Communications Services (PCS), Voice Mail, etc. (See COC TYPE) |  |  |
| SS | — | Switching Entity STATUS indicator (See D, E, M or -). A blank indicates the record is currently in effect. (See STATUS) |  |  |
| SSC | — | Special Service Code (SSC) is used in conjunction with the COC TYPE field to further identify special services provided by a Destination Code (NXX) record. (LERG6/9, Section 4/7). Allowable codes are: A = INTRALATA Use Only B = Paging Services ** C = Cellular Services ** I = Pseudo 800 Service Code J = Designates that this NXX has an extended/expanded local Calling scope. It is advisable to refer to the state tariffs for the Rate Center associated with this NXX. M = Local Mass Calling Code* N = Not Applicable O = Other (Explanation on notes line of BIRRDS on-line screens only) R = Two-way Conventional Mobile Radio** S = Miscellaneous Services (e.g., non-500 PCS, Voice Mail, etc.) T = Time* W = Weather* X = Service Provider requests Local Exchange Company IntraLATA Special Billing Option X indicates the dedicated code is assigned to a Service Provider who has requested a LEC IntraLATA special billing option on a LATA-wide basis. IntraLATA toll calls originating from LEC wireline subscribers are billed to the Service Provider as specified by state tariffs. A B, C or R entry, or combinations of B, and/or C, and/or R, and/or S entries should always accompany an “X” entry in the SSC field. Z = Service Provider requests SELECTIVE Local Exchange Company IntraLATA Special Billing Option Z indicates the dedicated code is assigned to a Service Provider who has requested a LEC IntraLATA special billing option on a SELECTIVE Exchange basis. IntraLATA toll calls originating from LEC wireline subscribers, in SELECTED Exchanges, are billed to the Service Provider as specified by state tariffs. A B, C, or R entry or combinations of B and/or C, and/or R, and/or S entries should always accompany a “Z” entry in the SSC field. |  |  |
|  |  | 8 = Puerto Rico and U.S. Virgin Islands codes * If different from Public Announcement System (976) code. Also, at this time, use of M, T, and W does not necessarily mean that the entire NXX is used solely for these services. # The digit “8” in the SSC field identifies those codes that are within Puerto Rico and the U.S. Virgin Islands and belong to either WATS Band 4 (for Florida and Rhode Island) or WATS Band 5 (for the remainder of the Continental U.S. portion of the NANP) and must be screened at the WATS originating screening office to pass calls from WATS Band 4 or 5. These codes must be reviewed each month in order to provide up-to-date originating WATS screening. ** Note that the information supplied by data providers regarding the “use” of an NXX may vary over time due to a company’s line assignments. Such changes are not necessarily conveyed to those maintaining this data. For example, a COC TYPE /SSC combination of EOC C (i.e., shared wireline/cellular) may technically become an EOC N (i.e., wireline only), but the data provider may not be aware that the initial inclusion of some cellular lines may have since changed. |  |  |
| STATE (ST) | — | This is the two-letter abbreviation that identifies a state, territory, province, or country (e.g. Caribbean NANP members). The two-letter code is that used in Telcordia COMMON LANGUAGE products. Outside the United States’ set of two-character state codes, the value used for a location may occasionally be different than two-letter codes used by other sources (e.g. foreign postal services). A listing of those codes that can appear in the LERG Routing Guide is provided on a table in section 1.3.1 State, Province, Island Code Table. |  |  |
| STATUS | — | Used in conjunction with an EFFDATE, the STATUS code indicates a specific type of activity that is to occur on that date. The STATUS label of S, in some cases, may be preceded by a letter (e.g. DS = Destination Code STATUS, SS = Switching Entity STATUS). - E = indicates that the specific record is to be “established” in the network on the associated EFFDATE. - M = indicates that a record in existence before associated “M” EFFDATE will have some data element(s) changed on the “M” EFFDATE. To determine the changing element(s) you must compare the data, field by field, to the preexisting state of the record (also provided in the LERG Routing Guide, usually the preceding line). - D = indicates that the specific record is to be “disconnected” from the network on the associated EFFDATE. - (b) (blank) = indicates the information provided is “current/active” at the time the LERG Routing Guide was produced. Note (LERG9): The STATUS code in LERG9 files is based on the NPA/NXX/SWITCH combination appearing under a given homed-to switch. An “E” or “D” STATUS means the combination is being established or disconnected relative to the homed-to switch. This does not necessarily mean that the NPA NXX and/or switch are disconnecting from the network. For example, an NPA NXX that may be moving from one switch to another (represented by a “M” (modify) STATUS in LERG6 files), where the “new” switch is under a different homed-to tandem than the “old” switch, will appear in LERG9 files with a “D” STATUS under the “old” homed-to tandem, and an “E” STATUS under the “new” homed-to tandem. |  |  |
| STP | — | Signaling Transfer Point. A packet switch in the CCS (Common Channel Signaling) network used for SS7 interconnection. (LERG7SHA, SOF Indicator) If the STP fields are populated (STPs occur in pairs; if one of the STP fields is populated, the other must also be populated), the switching entity has common channel signaling for IntraLATA use and, if it is also a CCS AC OFC (See definition in this glossary), it can be used by access purchasers for InterLATA common channel signaling. |  |  |
| STREET | — | Street portion of address in the switching entity address. (various files and reports) |  |  |
| Switch Identifier | — | See SWITCH |  |  |
| SWITCH | — | Also may be referenced as SW IDENT (paper LERG column heading), SWITCH ID, switch, switching entity, etc. The switch is “identified” through use of an 11-character descriptor/identifier. Aside from specific exceptions as denoted below, it is intended that this descriptor/identifier be an established COMMON LANGUAGE® Location Identifier (CLLIä) of the switch. (See CLN INDICATOR). Some of the reasons this field may not contain a CLLI are following: - For NXX’s in the NPA 500 and NPA 900, the SWITCH field is used to provide assignment, status, and routing information about the NXX (See Sections 1.9 and 1.10 for a more detailed explanation). - For NXX’s in the Caribbean and Bermuda NPAs, the SWITCH field is populated with SWCHxxUNKNO, where xx identifies one of the countries or territories in the Caribbean or Bermuda. - For NPA 710, the SWITCH field is populated with RTGETSAGENT (See Section 1.11). - For ODDBALL NXX’s, the SWITCH field is often populated with XXXXXXXXXXX as an indication that the “switch” involved may vary based on the type or service provided, due to carriers processing of a given NXX differently that other (e.g. network by network), etc. Note: In some cases the SWITCH may be a value that represents a “point of interface” (POI) and is technically not a “switch.” In such cases, it is expected that the data provided has provided a SWITCH as the ACTUAL SWITCH associated with the POI. Note: “Switch” is a term that has a basic definition of being a network element that will “switch” (e.g. route) call traffic to various other locations based on an assessment of the specifics of the transmission type, numbering protocol, etc., associated with the given call. However, the term has often become used in a loose sense in many areas of telecommunications. |  |  |
| SW 56 | — | A switched 56Kbps service, generically known as Public Switched Digital Service (PSDS), providing the end user (customer) with the ability to send and receive data at a speed of 56Kbps over the Public Switched Network (PSN), utilizing in-band signaling. (SOF Indicator) |  |  |
| TBP IND | — | See THOUSANDS BLOCK POOLING INDICATOR |  |  |
| TDM (TANDEM) | — | A tandem switch connects one trunk to another and serves as a trunk concentration and distribution function to minimize direct end office interconnection. It is an intermediate switch or connection between an originating switch and the final switch call destination. A tandem switch does not allow origination or termination of telephone calls. Tandems serve a designated geographic area consisting of specific rate centers. Tandem switches may perform one or more of the following functions or homing relationship: - Feature Group B Tandem (inter/intra LATA) - Feature Group C Tandem (inter/intra LATA) - Feature Group D Tandem (inter/intra LATA) - Operator Services Tandem (inter/intra LATA) Tandems that serve multi-LATAs have multiple appearances in the Tandem Homing Arrangement Section. (LERG 9, Section 7) |  |  |
|  |  | In some cases, homing arrangements are provided between a switching entity/POI and the following types of offices. These are not tandems but the homing relationships may be provided in the same manner (e.g. files, etc.) as actual switch/tandem homing: - Signaling Transfer Points - End Office Host - 800 SSP Office - Intermediate Office - Actual Switch/POI relationships |  |  |
| TDM FUNC CODES (TANDEM FUNCTION) | — | A single character field, which identifies the type of tandem function, performed and whether it performs this function for Originating, and/or Terminating traffic from or to a subtending End Office (LERG9, Section 7). The “field” itself may concurrently contain several of these specific values, as may be applicable since a given switch may perform various functions. A = Actual Switch B = Feature Group B Tandem C = Feature Group C Tandem D = Feature Group D Tandem (i.e., Equal Access tandem) F = Foreign Served Office H = Host I = Intermediate Office J = Feature Group B Intermediate Office K = Feature Group C Intermediate Office L = Feature Group D Intermediate Office M = Circuit Switched Data Tandem O = Operator Services Tandem S = Signaling Transfer Point T = IntraLATA Tandem U = Local Tandem 1 = Signaling Transfer Point 1 (STP1) 2 = Signaling Transfer Point 2 (STP2) 3 = Call Agent homing 4 = Trunk Gateway Homing 8 = 800 SSP |  |  |
| TEST LINE | — | This permits the data provider to identify the line number component of an NPA NXX BLOCK test line. This is an optional field at this time. Please be aware that some test lines may be operational only for a specified period and thereafter subject to later assignment to a subscriber. Assignment, use, and other aspects of test line numbers are largely a function of company-specific policies. (LERG6) |  |  |
| TEST LINE EXPIRATION DATE | — | When this field is populated, this indicates that the TEST LINE indicated for the NPA NXX BLOCK is expected to no longer be used as a TEST LINE after the indicated EXPIRATION DATE. Once this date has been passed, relative to the LERG product date, the EXPIRATION DATE as well as other TEST LINE information for the record will be blank in the LERG. (LERG6) |  |  |
| TEST LINE RESPONSE | — | This fields indicates the type of response that a call placed to the indicated TEST LINE should return: A = Announcement M = Milliwatt tone |  |  |
| THOUSANDS BLOCK POOLING INDICATOR | — | Permissible values are Y, N, S, I Y = Indicates that the NPA NXX has been identified to be part of a pool of NXX’s, within a given NPA, that are assigned 1000 lines at a time by the Pool Administrator (currently only applicable to the United States) to potentially different companies. (LERG6) N = Indicates that the NXX is not publicly pooled and that there is no information below the NXX level. S = Indicates that the NXX is not publicly pooled, but that the service provider has chosen, for purposes of Intra Service Provider (SP) Pooling, seven digit routing, or other reason, to show its fully assigned NXX to be “split” at the thousands block level. This can apply to any NPA within the NANP. I = Indicates the same as “S”, except that the Pool Administrator has been requested by the Code Holder to establish BIRRDS system controls regarding the “split” of its NXX into thousands blocks. This applies to only NPAs that are in the Pool Administrator’s inventory and currently applicable to just FCC regulated areas (U.S. and U.S. Territories). |  |  |
| TIME ZONE | — | This field designates the time zone(s) associated with the geographic coverage of the NPA (LERG3): 0 = Not applicable (e.g. NPA 800) 1 = Guam and the Commonwealth of the Northern Mariana Islands (CNMI) 2 = Hawaii 3 = Alaska/Yukon 4 = Pacific 5 = Mountain 6 = Central 7 = Eastern 8 = Atlantic 9 = Newfoundland (one and a half hours ahead of Eastern) |  |  |
| TOTAL LINES/VIEW | — | Identifies the TOTAL number of unique lines per VIEW ID. (LERG9) (See VIEW ID, VIEW LINE #) |  |  |
| TR DIG | — | Note: The following does not apply to NPA NXX records at the Thousands Block Level. TR DIG values associated with numeric block records will appear as NA (Not Applicable). These fields indicate the number of terminating digits (TR DIGs) to be outpulsed to a switching entity/POI or tandem when completing (terminating) a call (LERG6, Section 4). Note that this field, for numeric BLOCK IDS will always be ‘NA’. The EO (End Office) field indicates the number of terminating digits required if the call is to be directly routed to the “end office” at which the NPA NXX resides. The AT (Access Tandem) field indicates the number of terminating digits required if the routing of the call is via the primary access tandem associated in LERG7SHA, LERG9 (Section 7) with that end office. Both the EO field and the AT field must be populated. Blank entries are not allowed. An “NA” in either the EO or AT field means that the LSP provides for no direct trunking to the switching entity or point of termination (POT) in the LATA. The entries in the TR DIG fields depend, in part, on the entry in the COC TYPE field. Currently, the following COC TYPE fields are in use: COC TYPE Description ------ --------------- ATC Access Tandem Code (0/1XX) CDA Customer Directory Assistance (Line numbers within the 555 NXX are assigned by the North American Numbering Plan Administration) EOC End Office Code PLN Planned (non-routable) PMC Public Mobile Carrier (Type 2 Interconnected) RCC Radio Common Carrier (Type 1 Interconnected) SIC Special 800 Service Code SP1 Service Provider – Miscellaneous Service (Type 1 Interconnected) SP2 Service Provider – Miscellaneous Service (Type 2 Interconnected) TST Standard Plant Test Code These COCTYPEs can be grouped as follows: (Group 1) All except TST and ATC. This group includes CDA, EOC, PMC, RCC, PLN and SIC. (Group 2) TST, and (Group 3) ATC. Each of these groups is discussed below. |  |  |
| TR DIG (cont) | — | Group 1: All COCTYPEs Except TST and ATC EO Field: In many cases the required number of digits will be 7 (NXX + Line Number). However, for non-conforming end offices (Feature Group B and C access) and cross-boundary offices, the number of terminating digits required could vary from as few as 4 (just the line number) to as many as 10 (the NPA+NXX+line number). As NPA overlays become more prevalent and as a switching entity may cover a broader geographic area, the need to require 10 digits will expand. Additionally, PMC COCTYPEs may have an “NA” in the EO field and CDA COCTYPEs may have a “0” in the EO field. AT Field: This field will be populated with either 7 or 10. Only seven terminating digits (NXX + Line Number) are needed if the NPA-NXX being terminated is the same as the Home NPA of the Access Tandem. This case exists when the Access Tandem serves a single NPA, since no ambiguity exists. Ten terminating digits (NPA + NXX + Line Number) are needed if the terminating NPA is different from the Home NPA of the Access Tandem. This case exists when the Access Tandem serves two or more NPAs. The three-digit terminating NPA is needed to resolve the ambiguity between an NXX, which is assigned in two or more NPAs, served by a single Access Tandem. Group 2: COC TYPE of TST Test codes labeled with a COC TYPE of TST are special use codes. The required number of terminating digits in either the EO or the AT field could range from as few as 3 to as many as 10. Normally, the LERG Routing Guide will reflect the maximum, which will usually be 10, and the correct number to use must be determined through one-on-one discussions. Group 3: COC TYPE of ATC When the COC TYPE is ATC, the call either will be terminated directly at an Operator Service Tandem or will be terminated at an Exchange Carrier's Access Tandem with terminating digits that will identify the Operator Service Tandem to which the call is to be terminated. EO Field: This field will always be NA, since Operator Service Access Tandem Codes, designated by the COC TYPE of ATC, will never terminate at an End Office. AT Field: If the call is being terminated at an Operator Service Tandem, only the Operator Service Code needs to be outpulsed. Operator Service Codes are 3, 4 or 5 digits, and vary by company. If the call is being terminated at an Exchange Carrier's Access Tandem and needs to be further routed to an Operator Service Tandem, the three-digit Access Tandem Code (ATC) must be outpulsed as well as the 3-, 4- or 5-digit Operator Service Code. Therefore, depending on the company and the type of tandem at which the call is terminating, the entry in this field may be as low as 3 or as high as 8. |  |  |
| TRUNK GATEWAY | — | An access gateway provides the line side interface between the Public Switched Telephone Network (PSTN) and the Voice over Packet (VoP) core network. The Trunk Gateway provides an interface between the PSTN digital trunk facility and the VoP core network. (LERG7, LERG7SHA) |  |  |
| V&H Coordinates | — | For a more complete definition see HC (Horizontal Coordinate), VC (Vertical Coordinate) and MAJOR RC VC / HC. |  |  |
| VC (Vertical Coordinate) | — | For a more complete definition, see HC (Horizontal Coordinate). |  |  |
| VERSION | — | In Insert files only. This should always be an ‘A’; however, the field may, on occasion, contain a ‘B’, etc. You should only have one “version” letter for a given Activity Date. Versions exist should updates to issued activity be required for LERG One-Day Changes users but are not directly pertinent to monthly LERG “Insert” users. (Insert files) |  |  |
| VIEW ID | — | When applicable in data files, this associates an “ID” number with the given “view” of a record. In the LERG9 files, data for a given key/effective date may be broken down further (e.g. subtending switch / NPA NXX combinations). View ID is then intended as a means to identify the “continuation” of the given information across multiple lines. Note that the VIEW ID value may vary month-to-month for a given key and is a tracking vehicle, not specifically a data element associated with the key. VIEW IDs are sequentially assigned based on the actual sort order in the final version of the LERG Routing Guide .DAT files. (LERG9) (See VIEW LINE #, TOTAL LINES/VIEW) |  |  |
| VIEW LINE # | — | VIEW LINE # tracks the sequential number of “lines” of data for a given VIEW ID. (LERG9) (See VIEW ID, TOTAL LINES/VIEW) |  |  |
| W RESELLER | — | Wireless Reseller - a company that leases a block of numbers, in bulk, from wireless carriers, for purposes of resale to customers. Resellers often lease facilities as well. (See CATEGORY) |  |  |
| WIRELESS | — | Companies sanctioned to provide local exchange service on a non-wireline (wireless) basis pursuant to FCC Regulations Part 22. This includes cellular companies, paging (beepers) companies, and Improved Mobile Telephone System (IMTS) (radio). (See CATEGORY) |  |  |
| WIRELESS OFC (Wireless Office) | — | An interconnection point that provides either or both, originating dial tone and a terminating service to a Wireless subscriber. (SOF Indicator) |  |  |
| WIRELESS TDM (Wireless Tandem) | — | A tandem supporting a wireless switch for originating and/or terminating traffic. (SOF Indicator) |  |  |
| X.75 GATEWAY | — | Indicates the switch provides interconnection service to interexchange packet data carriers via the X.75 protocol. (SOF Indicator) |  |  |
| ZIP / POSTAL CODE | — | Identifies the ZIP code associated with an address in the United States. This would be the Postal Code used in addresses of non-US areas in the NANP. |  |  |
| 800 SSP | — | A switching entity that can launch database queries for originating 800 traffic. This entity can also initiate 800 database queries for other switches that do not have the 800 SSP functionality if the 800 call routes to this switch. (SOF Indicator, LERG7SHA, SWITCH.) |  |  |
| 911 Tandem | — | This Switching Office Functionality (SOF) indicator is used to identify a switch that serves as a 911 Tandem. A 911 Tandem provides trunk-to-trunk connections between end offices and a switch that services Public Safety Answering Points (PSAPs). The specific municipalities and geographic area that the associated 911 service may cover, PSAPs, etc., are not addressed in the LERG Routing Guide at this time. (SOF Indicator, LERG15) |  |  |

### 1.3.1	State, Province, Island Code Table*

| Canada - |  | United States - |  | United States (cont'd) |  |
| --- | --- | --- | --- | --- | --- |
| Alberta | AB | Alabama | AL | Montana | MT |
| British Columbia | BC | Alaska | AK | Nebraska | NE |
| Manitoba | MB | Arizona | AZ | Nevada | NV |
| New Brunswick | NB | Arkansas | AR | New Hampshire | NH |
| Newfoundland** | NF | California | CA | New Jersey | NJ |
| Northwest Territory | NT | Colorado | CO | New Mexico | NM |
| Nova Scotia | NS | Connecticut | CT | New York | NY |
| Nunavut Territory** | VU | Delaware | DE | North Carolina | NC |
| Ontario | ON | District of Columbia | DC | North Dakota | ND |
| Prince Edward Island | PE | Florida | FL | Ohio | OH |
| Quebec** | PQ | Georgia | GA | Oklahoma | OK |
| Saskatchewan | SK | Hawaii | HI | Oregon | OR |
| Yukon Territory | YT | Idaho | ID | Pennsylvania | PA |
|  |  | Illinois | IL | Rhode Island | RI |
| Mexico - | MX | Indiana | IN | South Carolina | SC |
|  |  | Iowa | IA | South Dakota | SD |
| Islands - |  | Kansas | KS | Tennessee | TN |
|  |  | Kentucky | KY | Texas | TX |
| American Samoa**** | AS | Louisiana | LA | Utah | UT |
| Anguilla | AI | Maine | ME | Vermont | VT |
| Antigua | AN | Maryland | MD | Virginia | VA |
| Bahamas | BA | Massachusetts | MA | Washington | WA |
| Barbados | BD | Michigan | MI | West Virginia | WV |
| Bermuda | BM | Minnesota | MN | Wisconsin | WI |
| British Virgin Islands | BV | Mississippi | MS | Wyoming | WY |
| Cayman Islands | CQ | Missouri | MO |  |  |
| CNMI (N. Marianas)*** | NN |  |  |  |  |
| Dominica | DM |  |  |  |  |
| Dominican Republic | DR |  |  |  |  |
| Grenada | GN |  |  |  |  |
| Guam | GU |  |  |  |  |
| Jamaica | JM |  |  |  |  |
| Montserrat | RT |  |  |  |  |
| Puerto Rico | PR |  |  |  |  |
| St. Kitts & Nevis | KA |  |  |  |  |
| St. Lucia | SA |  |  |  |  |
| St. Vincent | ZF |  |  |  |  |
| Trinidad & Tobago | TR |  |  |  |  |
| Turks & Caicos | TC |  |  |  |  |
| US Virgin Islands | VI |  |  |  |  |

*       Abbreviations are based on COMMON LANGUAGE assignments and may not correlate to other sources (e.g. postal, ISO, etc.)

**     Canada Post abbreviations are:  Newfoundland/Labrador, NL; Nunavut Territory, NU; Quebec, QC

***   USPS abbreviation is MP

**** Regarding American Samoa, AS is the USPS abbreviation; AM (not used) is the COMMON LANGUAGE abbreviation

## 1.4	Special Routing/NXX Codes

The following codes are identified herein to provide some added information on their use.  These “codes,” unique NPAs, NXX’s, or 0/1XX codes, exist for specific purposes in the telecommunications industry.  Note that some of these codes are based on historical usage and that definitive industry documentation confirming the universality of use of some of these assignments may not exist.  Further information may be available, in some cases, at www.nanpa.com and www.cnac.ca.  Their use, as described below, may vary by individual service provider, NPA, state/province/country, etc.

### 1.4.1	Special Plant Test Codes (1XX)

- This section lists Plant Test routing codes in the 1XX series.  Note that other Plant Test or general “test” codes may exist as specific NXX’s, other specific 0/1XX codes, or specific line numbers within an NXX.  They may locally defined by a service provider, may be used for an interim period (e.g. NPA split test numbers), etc.

- ASSIGNED:

100 - Balance Termination/Quiet

101 - Testboard

102 - Milliwatt Supply

103 - Signaling Test Termination

104 - Two-Way Transmission and Noise Checking

105 - Automatic Transmission Measuring

106 - CCSA LOOP - Around Transmission Test

107 - Par Meter Generator

108 - DS-0 Loopback Test System

109 - ECHO Canceller

### 1.4.2	Service Codes

- SERVICE CODES (N11)

- These codes are dialed as the noted three digits.  Details regarding the implementation, routing, and specific use of the following may vary to some degree by NPA, state/province/country, etc., since the actual use, as well assignment, of some of these codes fall under the auspices of varying jurisdictional and regulatory bodies (e.g. state utility commissions).  However, the following are generally accepted designations, especially with the United States, its Territories, and Canada.  Note that N11 codes are identified in LERG data files that involve NPA NXX listings and are classified as a component of “Oddball” codes.  N11 cannot be assigned as NPAs since obvious conflicts would arise.

211 - Community Support and Referral Services

311 - Non-Emergency Government Services

411 - Telco Directory Assistance

511 - Government provided Traffic/Travel/Road condition report (US)

(reserved in Canada)

611 - Telco Repair Service

711 - Telecom Relay Operator for Hearing/Speech Impaired

811 - Telco Business Office

- 911 - Emergencies (Police, Fire, Ambulance, etc.)

- SERVICE ACCESS CODES (SACs)

The following can be considered special NPAs in that they have been assigned for non-geographic purposes.  NXX and line numbers are assigned by the appropriate party (i.e., a carrier or a governmental agency) that has been identified to have responsibility for either the entire SAC (e.g. 710) or is assigned specific NXX’s (e.g. 900).  Toll free are assigned a number at a time, from a common database, by companies serving as Toll-free service providers.  Numbers in the format N00, N22 … N88 (excepting 555) that are not addressed by the table below are considered SACs, but do not currently have an assigned function.

- ASSIGNED:

456 – International Inbound***

500 – Personal Communications Services*

600 – Canadian Services

700 – Interexchange Carriers

710 – U.S. Federal Government Services*

800 – Toll Free Service**

900 – 900 Service*

* Information regarding specific assignments in 500 and 900 appear in the LERG NPA NXX data files, as does the primary NXX for 710.

** 8XX where XX = 22, 33, …, 88 also exist for Toll Free Service.  Through June, 2003, 88, 77, 66 are in use while the remaining are reserved for future use.

***  The assignments for 456 (as existed on 12/1/03) are as follows.  Assignments in 456 have been essentially stagnant.

NXX		CARRIER		DATE ASSIGNED

226		Teleglobe Canada		08/30/93

288		AT&T				08/15/97

289		AT&T				08/23/93

333		Startec, Inc.			04/11/95

624		MCI				08/16/93

640		Sprint				10/05/93

741		STSJ				08/26/93

808		Hawaiian Tel			02/25/94

- RESERVED CODES

The following can be considered reserved NPAs in that they have been set aside for potential future use as noted:

N9X – this grouping of numbers has been set aside to address, under current scenarios, how to deal with the need to expand the current 10 digit NANPA (N=2-9, X=0-9)

37X, 96X – this grouping of numbers have been set aside should there be need for a full series of ten consecutive numbers be needed for any new service, technological, etc. reasons that may develop as the industry evolves.

### 1.4.3	NANP Universal Central Office Codes (NXX’s)

- The following NXX codes (note that N11 was addressed in the previous section) have unique functionalities within the United States and Canada, at a minimum, as described below.  Note that due to the ever evolving nature of the industry, isolated cases may exist, on a grandfathered basis, where within a given NPA, one or more of the following, primarily within 958, 959, or 976, may be assigned (or may have been assigned at some point) to a service provider as a standard NXX.

- ASSIGNED:

555 - Toll Directory Assistance

Note: LERG Routing Guide information relative to NXX 555 refers to the Directory Assistance aspects of 555 (i.e., line number 1212).  Other lines in the 555 NXX are assigned by the North American Numbering Plan Administration (www.nanpa.com) based on 555 Line Number Assignment Guidelines created by the telecommunications industry and available at www.atis.org (INC).

700 - IntraLATA PIC Validation

Note: 700 may be assigned to a single service provider, but only with the understanding that line number 4141 is protected and is only used for its intended purpose.  In 1996, line number 700-4141 was established in each NPA as access for verification of a customer’s intraLATA carrier.

950 - FGB Access Code - The 950 code plus the CIC make a Carrier Access Code (CAC): 950-XXXX.

958 - Plant Test

959 - Plant Test

976 - Information Delivery Service only

### Operator Service Codes

The following are codes used historically in conjunction with operator-to-operator calls and are those associated, in whole or part, with ATC (Access Tandem Codes) in the LERG.  With the many technological advances, increased number of carriers, third-party operator services, etc., that have occurred over the past few decades, the need for and use of these codes have diminished.  However, they are still used in many cases.

121	-	Inward

131	-	Directory Assistance

141	-	Route Desk

160	-	IOC Access

181, 11881	-	Toll Station

1150, 11501	-	Universal or Coin Callback

1151, 11511	-	Conference

1152, 11521, 11821	-	Mobile

1153, 11531, 11831	-	Marine

1154, 11541	-	Toll Terminal

1155, 11551	-	T&C Callback (Time and Charges)

1156, 11561	-	Hotel Callback

1157, 11571	-	IOTC Access Trunk

1158, 11581	-	Inward Completion Assistance

1159, 11591	-	Inward Busy Line Verification

1160, 11601	-	Calling Card Validation (Dial Pulse Equipment)

1161, 11611	-	Calling Card Validation (DTMF)

1162, 11621	-	Calling Card Validation (MF- Multifrequency Equipment)

1.5	Signaling System 7 (SS7) Network Codes

The Signaling System 7 (SS7) network codes are assigned and administered by Telcordia Technologies acting as maintenance agent for Committee T1 under a contract between Telcordia Technologies and the Alliance for Telecommunications Industry Solutions (ATIS).  Questions regarding these assignments should be referred to:

- Telcordia Technologies

8 Corporate Place

Room PYA-3E121

Piscataway, NJ  08854

732-699-4204

Committee T1 developed and maintains American National Standard T1.111-1988.  Chapter 8 (T1.111.8) of the standard, titled “Numbering Signaling Point Codes” describes the format for signaling point codes and the assignment procedures for network codes.  Companies wishing to obtain SS7 point codes for their company can also obtain a CCS Point Code application form from www.trainfo.com (documents) or directly from www.ss7pcadmin.com.

The assignments themselves are provided in the paper LERG Routing Guide (see Section 3.4), and are in the LERG4 data file.

- Also, see the Glossary for NETWORK, CLUSTER, and MEMBER.

1.6	Vertical Service Codes

### 1.6.1	Introduction

Vertical Service Codes (VSCs) are customer-dialed codes used to access existing and future vertical services (e.g., Call Forwarding).  VSCs are standardized in the format *XX and *2XX for touch-tone telephones and 11XX and 112XX for rotary telephones with X = 0 to 9.  The North American Numbering Plan Administration (NANPA) assigns VSCs upon request of service providers such as Local Exchange Carriers (LECs) interexchange carriers, commercial mobile radio service (CMRS) providers etc., using guidelines agreed to by the industry and specified in document INC 96-0802-015.  VSCs are assigned on a national basis; i.e., a *XX or *2XX code assignment is intended to be used for the assigned service anywhere within the North American Numbering Plan (NANP) area.  Contained herein is the current list of VSCs that have been assigned by NANP administration, then followed by a brief definition of each service.  The list of assigned VSCs will be updated periodically to reflect new assignments.

The primary objective in the assignment of VSCs by NANP administration is to standardize the access codes for services that are deemed universal or national in scope such that users may dial the same access code for a specific service regardless of where or by whom the service is being offered.

Requests for assignment of vertical service codes, or information on VSC assignments should be directed to:

NANPA

46000 Center Oak Plaza

Sterling, VA  20166

### 1.6.2	Vertical Service Code Assignments

Table A lists the Vertical Service Codes (VSCs) that have been assigned by NANPA.  The *3 range on this list has been reserved for future expansion of VSCs; therefore, VSCs will not be assigned in this range.  A small range of codes (*94 to *99) has been set aside for local use (i.e., to be used locally by carriers as appropriate), and no services will be assigned codes within this range by NANP administration.

- TABLE  A

| Code | Service Assignment |
| --- | --- |
| *00 | Inward Voice Activated Services (English) |
| *01 | Inward Voice Activated Services (French) |
| *02 | Deactivation/Activation of In–Session Activation(ISA) on a per line basis |
| *03 | Deactivation of In–Session Activation (ISA) on a per call basis |
| *04 | Unassigned |
| *05 | Unassigned |
| *06 | Unassigned |
| *07 | Unassigned |
| *08 | Unassigned |
| *09 | Unassigned |
| *1X | Unassigned |
| *2X | Reserved for expansion to a three–digit numeric format (*2XX) |
| *228 | Over–the–Air Service Provisioning |
| *3X | Reserved for expansion to a three–digit numeric format (*3XX) |
| *40 | Change Forward–To Number for Customer Programmable Call Forwarding Busy Line |
| *41 | Six–Way Conference Calling Activation |
| *42 | Change Forward–To Number for Customer Programmable Call Forwarding Don't Answer |
| *43 | Drop last member of Six–Way Conference Call |
| *44 | Voice Activated Dialing |
| *45 | Voice Dialing Extended Dial Tone |
| *46 | French Voice Activated Network Control |
| *47 | Override Feature Authorization |
| *48 | Override Doe Not Disturb |
| *49 | Long Distance Signal |
| *50 | Voice Activated Network Control |
| *51 | Who Called Me? |
| *52 | Single Line Variety Package (SVP) – Call Hold |
| *53 | Single Line Variety Package (SVP) – Distinctive Ring B |
| Code | Service Assignment |
| *54 | Single Line Variety Package (SVP) – Distinctive Ring C |
| *55 | Single Line Variety Package (SVP) – Distinctive Ring D |
| *56 | Change Forward–To Number for ISDN Call Forwarding |
| *57 | Customer Originated Trace |
| *58 | ISDN Multi Button Key Set (MBKS) Manual Exclusion Activation |
| *59 | ISDN Multi Button Key Set (MBKS) Manual Exclusion Deactivation |
| *60 | Selective Call Rejection Activation |
| *61 | Distinctive Ringing/Call Waiting Activation |
| *62 | Selective Call Waiting |
| *63 | Selective Call Forwarding Activation |
| *64 | Selective Call Acceptance Activation |
| *65 | Calling Number Delivery Activation |
| *66 | Automatic Callback Activation |
| *67 | Calling Number Delivery Blocking/Calling Identity Suppression |
| *68 | Call Forwarding Busy Line/Don't Answer Activation |
| *69 | Automatic Recall Activation |
| *70 | Cancel Call Waiting |
| *71 | Usage Sensitive Three–way Calling |
| *72 | Call Forwarding Activation |
| *73 | Call Forwarding Deactivation |
| *74 | Speed Calling 8 – Change List |
| *75 | Speed Calling 30 – Change List |
| *76 | Advanced Call Waiting Deluxe |
| *77 | Anonymous Call Rejection Activation |
| *78 | Do Not Disturb Activation |
| *79 | Do Not Disturb Deactivation |
| *80 | Selective Call Rejection Deactivation |
| *81 | Distinctive Ringing/Call Waiting Deactivation |
| *82 | Line Blocking Deactivation |
| *83 | Selective Call Forwarding Deactivation |
| *84 | Selective Call Acceptance Deactivation |
| *85 | Calling Number Delivery Deactivation |
| *86 | Automatic Callback Deactivation |
| *87 | Anonymous Call Rejection Deactivation |
| *88 | Call Forwarding Busy Line/Don't Answer Deactivation |
| *89 | Automatic Recall Deactivation |
| *90 | Customer Programmable Call Forwarding Busy Line Activation |
| *91 | Customer Programmable Call Forwarding Busy Line Deactivation |
| *92 | Customer Programmable Call Forwarding Don't Answer Activation |
| *93 | Customer Programmable Call Forwarding Don't Answer Deactivation |
| *94 | Reserved For Local Assignment |
| *95 | Reserved For Local Assignment |
| *96 | Reserved For Local Assignment |
| *97 | Reserved For Local Assignment |
| *98 | Reserved For Local Assignment |
| *99 | Reserved For Local Assignment |

### 1.6.3	Definitions of Vertical Service Code Assignments

Table B lists the above services in alphabetical order and provides a brief definition of each service.  Underneath the service name are the *XX service activation and deactivation codes from Table A.  These definitions are not intended to be exhaustive, but have been provided to help service providers identify similar services that might be offered within their companies under different names.

- TABLE B

| SERVICE | DEFINITION |
| --- | --- |
| Advanced Call Waiting Deluxe *76 | Allows a subscriber to specify, in advance of incoming calls, the termination treatment for calls that arrive while the subscriber is engaged in another conversation |
| Anonymous Call Rejection *77 Activation *87 Deactivation | Allows customers to reject calls from parties who have a privacy feature that prevents the delivery of their calling number to the called party |
| Automatic Callback *66 Activation *86 Deactivation | Allows a subscriber to automatically place a call to the last station called by the subscriber, when that station becomes idle. |
| Automatic Recall *69 Activation *89 Deactivation | Allows a subscriber to automatically place a call to the last station that called the subscriber, when that station becomes idle. |
| Call Forwarding *72 Activation *73 Deactivation | Allows a subscriber to redirect calls intended for his/her station (base station) to another (remote station.) |
| Call Forwarding Busy Line/Don't Answer *68 Activation *88 Deactivation | Allows a subscriber to forward calls intended for the subscriber's busy line, or idle line after a predetermined number of rings, to another directory number entered by the subscriber at the time of activation. |
| Calling Number Delivery *65 Activation *85 Deactivation | Provides the subscriber with the directory number of the calling party during the ringing cycle. |
| Calling Number Delivery Blocking/Calling Identity Suppression *67 | Allows the subscriber to temporarily change the permanent public/private status indicator of his/her director number and thus control its availability to the called party. Alternate definition (Calling Identity Suppression): Allows the subscriber to temporarily suppress delivery of both the caller's directory number and the calling name, independent of permanent status. |
| Cancel Call Waiting *70 | Provides the subscriber the ability to disable the Call Waiting feature for the duration of a telephone call. |
| Change Forward-To Number for Customer Programmable Call Forwarding Busy Line *40 | Access Code followed by directory number is used to change the forwarded-to number for Call Forwarding Busy Line (CFBL). The state of CFBL is not changed when this access code is used. This feature will utilize the activation codes of *90 and deactivation code *91 with the following exceptions: activation will not require/allow the identification of a forwarded-to directory number and deactivation will not clear the forwarded-to directory number. |
| Change Forward-To Number for Customer Programmable Call Forwarding Don't Answer *42 | Access Code followed by directory number is used to change the forwarded-to number for Call Forwarding Don't Answer (CFDA). The state of CFDA is not changed when this access code is used. This feature will utilize the activation codes of *92 and deactivation code *93 with the following exceptions: activation will not require/allow the identification of a forwarded-to directory number and deactivation will not clear the forwarded-to directory number. |
| Change Forward-To Number for ISDN Call Forwarding *56 | Access code followed by directory number is used to change the Forward-To number for Call Forwarding Variable feature button. The state of Call Forwarding Variable feature button is not changed when this access code is utilized. |
| Customer Originated Trace *57 | Provides the recipient of an obscene, harassing, or threatening call the ability to request a trace of the last call received. |
| Customer Programmable Call Forwarding Busy Line *90 Activation *91 Deactivation | Allows subscriber of the feature to forward calls intended for the subscriber's busy line to another directory number entered by the subscriber at the time of activation. Deactivation will clear the forwarded-to directory number. |
| Customer Programmable Call Forwarding Don't Answer *92 Activation *93 Deactivation | Allows a subscriber of the feature to forward calls intended for the subscriber's idle line, after a predetermined number of rings, to another directory number entered by the subscriber at the time of activation. Deactivation will clear the forwarded-to directory number. |
| Deactivation/Activation of In-Session Activation (ISA) on a per line basis *02 Activation 03 Deactivation | Allows a subscriber to deactivate or activate (i.e., toggle) the In-Session Activation feature on a per line basis. ISA is a feature that gives the caller a menu of call completion services using voice prompts when the call encounters a busy or no-answer condition. |
| Distinctive Ringing/Call Waiting *61 Activation *81 Deactivation | Allows the subscriber to have incoming calls from a limited number of calling parties identified using distinctive alerting treatment. |
| Do Not Disturb *78 Activation *79 Deactivation | Provides the subscriber the opportunity of having all calls intercepted by the Central Office switch whenever the line is programmed for Do Not Disturb. The calling party will receive a message indicating the station is in Do Not Disturb condition. |
| Drop last member of Six-Way Conference Call *43 | Provides the subscriber establishing a six-way conference to terminate the last party added to the call. This frees the port for an additional party when the last party was not reachable. |
| French Voice Activated Network Control *46 | Provides the subscriber access to Voice Activated Network Control (VANC) via the French language. Subscribers will dial this code to access VANC so that they can say a name or command in French that will be activated, deactivated or provide access to a service, e.g., Call Forwarding, Call Trace, etc. |
| Inward Voice Activated Services *00 English *01 French | IVAS enables a subscribing business to provide automated voice activated routing for inbound English- or French-speaking calls (i.e., separate codes for the same service in each language). IVAS will initially consist of the following services: - Voice Activated Premier Dialing (VAPD) that allows customers to contact subscribing businesses by speaking the business name or service. - Voice Activated Blue Pages (VABP) that allows customers to request access to government services. - Voice Activated Auto Attendant (VAAA) that provides enhancements to Auto Attendant applications by providing a voice recognition interface in place of Tough Tone. - Voice Activated Audio Text (VAAT) provides users ability to request specific information from a business. - Voice Activated Interactive Voice Response (VAIVR) that allows the caller to interact with a subscriber's specific application in a prescribed manner. |
| ISDN MBKS Manual Exclusion *58 Activation 59 Deactivation | Access code allows a Multibutton Key Set (MBKS) user or an analog set user whose telephone number is shared on another ISDN MBKS to inhibit other stations from picking up a call on hold or bridging onto a call that is active at the station. |
| Line Blocking Deactivation *82 | Allows a caller to dial a delivery feature access code before dialing a complete telephone number to temporarily override the presentation status of both the caller's directory number and the calling name. If the caller enters the delivery code, then the calling identity presentation status will be shown as “public” for both caller directory number and calling name. |
| Long Distance Signal *49 | Extended period activation/deactivation (toggle) of basic 1FR/1MR long distance signal ringing/call waiting tones. |
| Override Feature Authorization *47 | Allows a subscriber to override a Feature Authorization activated on a line that restricts 1+ calls from that line. Feature Authorization may be overridden by dialing *47 and a Personal Identification Number (PIN) and then dialing a 1+ call after receiving a second dial tone. |
| Override Do Not Disturb *48 | Allows a subscriber to override the Do Not Disturb feature that has been activated on a line. After receiving a message indicating the station is in a Do Not Disturb condition, the subscriber may override the condition by dialing *48 and then a Personal Identification Number (PIN) thus allowing the call to be completed in the normal manner. |
| Over-the-Air Service Provisioning *228 | OTASP will enable the Service Provider to Activate a potential service to a subscriber's wireless unit by downloading over the air required parameters, such as phone numbers, into the handset. Activation of the OTASP code, followed by supplemental digit strings, also provides the ability to securely load an Authentication Key into a subscriber's wireless phone which is used to confirm and validate the identity of the wireless handset. |
| Selective Call Acceptance *64 Activation *84 Deactivation | Provides the subscriber the ability to block calls from all but a predetermined list of directory numbers specified by the subscriber. Unaccepted callers may receive an announcement or be routed to a predetermined directory number. |
| Selective Call Forwarding *63 Activation *83 Deactivation | Allows the subscriber to have incoming calls from a limited number of calling parties forwarded to a pre-specified remote station. |
| Selective Call Rejection *60 Activation *80 Deactivation | Allows the subscriber to have incoming calls from a limited number of calling parties rejected by the terminating switching system. |
| Selective Call Waiting *62 | Provides the subscriber the ability to receive a Call Waiting signal when called from a predetermined list of directory numbers specified by the subscriber. Callers not on the predetermined list will receive busy tone. |
| Single Line Variety Package (SVP) - Call Hold *52 | Gives the subscriber the capability of placing a call on hold so that the call may be continued from another extension. |
| Single Line Variety Package (SVP)- Distinctive Ring B *53 | Allows a subscriber to select, by way of distinctive ringing, the particular person or extension that the subscriber wishes to alert. |
| Single Line Variety Package (SVP)- Distinctive Ring C *54 | Allows a subscriber to select, by way of distinctive ringing, the particular person or extension that the subscriber wishes to alert. |
| Single Line Variety Package (SVP)- Distinctive Ring D *55 | Allows a subscriber to select, by way of distinctive ringing, the particular person or extension that the subscriber wishes to alert. |
| Speed Calling *74 Speed Calling 8-Change List *75 Speed Calling 30-Change List | Allows a subscriber to assign his/her own speed calling codes directly and immediately from his/her own telephone by dialing a change speed calling list access code, an abbreviated code, and a new telephone number. |
| Usage Sensitive Three-way Calling (*71) | Allows a subscriber, by dialing an access code, to request the capability of adding a third party to the two-way connection that is established by subsequent dialing. |
| Voice Activated Dialing *44 | Access to the Voice Activated Dialing (VAD) directory. Customers will dial this code to access their VAD directory in order to add, delete, or review the names and numbers. |
| Voice Activated Network Control *50 | Access to Voice Activated Network Control (VANC). Customers will dial this code to access VANC so that they can say a name or command that will be activated, deactivated or to access a service. |
| Voice Dialing Extended Dial Tone *45 | Extend dial tone for Voice Activated Dialing (VAD). Customers will dial this code to extend the length of time in which dial tone is heard after going off-hook so that various Customer Premise Equipment (CPE, e.g., FAXes and modems) will work properly. |
| Who Called Me? (*51) | Provides the subscriber with the directory number date, and time of unanswered calls. |

## 1.7	Carrier Identification Codes (CICs)

The Carrier Identification Codes (CICs) are assigned and administered by the North American Numbering Plan Administration (NANPA).  The CICs are assigned to entities purchasing Feature Group B (FG B) or Feature Group D (FG D) access service.  FG B and FG D CICs are assigned from separate pools of numbers so that there are 10,000 numbers available as FG B CICs and a separate pool of 10,000 numbers available as FG D CICs.  CICs are used when dialing to reach the entity to which the CIC is assigned.  The Feature Group B CICs are used as the XXXX in the format of 950-XXXX.  The Feature Group D CICs are used in the format of 101-XXXX.

CICs represent an industry resource and the North American Numbering Plan Administration is charged with their conservation.  The assignment of CICs and their conservation is performed under INC document INC 95-0127-006, Carrier Identification Code Assignment Guidelines, Revised September, 1996 (Formerly ICCF 92-0726-002).  A copy of the Guidelines can be obtained at www.atis.org (INC).

A complete list of CIC Assignments can be obtained from the NANPA web site, www.nanpa.com.  Questions can be directed to:

NANPA

46000 Center Oak Plaza

Sterling, VA  20166

1.8	Automatic Number Identification (ANI) Information Indicator (II) Digits Codes

This section lists and defines the use of ANI Information Indicator Digits Codes.  These are two-digit codes which precede the 7 or 10-digit directory number (DN) of the calling line and inform exchange and interexchange carriers about the type line that is originating the call, any special characteristics of the billing number, or certain service classes.  The two-digit codes and the directory numbers are part of the signaling protocol in equal access offices and are outpulsed by the originating switching system to the receiving office for billing, routing, or special handling purposes.  Further explanation of the signaling protocol can be found in TR-NPL-000258 and TR-NPL-000275.

ANI Information Indicator Digits Codes are assigned by the North American Numbering Plan Administrator (NANPA) at the request of industry forums/associations that have reached consensus that a code is required for a specific application.  Further information on the assignment guidelines can be found in AL-87/05-007.  Products of the Telcordia Routing Administration (TRA) (e.g., LERG Routing Guide) do not identify these codes by switch.

For further information regarding ANI II assignments please refer to www.nanpa.com.  Questions regarding these codes can also be referred to:

NANPA

46000 Center Oak Plaza

Sterling, VA  20166

| ANI II |  | DEFINITION |
| --- | --- | --- |
| 00 | – | Plain Old Telephone Service (POTS) - non-coin service requiring no special treatment. |
| 01 | – | Multiparty line (more than 2) - ANI cannot be provided on 4 or 8 party lines. The presence of this “01” code will cause an Operator Number Identification (ONI) function to be performed at the distant location. The ONI feature routes the call to a CAMA operator or to an Operator Services System (OSS) for determination of the calling number. |
| 02 | – | ANI Failure - the originating switching system indicates (by the “02” code), to the receiving office that the calling station has not been identified. If the receiving switching system routes the call to a CAMA or Operator Services System, the calling number may be verbally obtained and manually recorded. If manual operator identification is not available, the receiving switching system (e.g., an InterLATA carrier without operator capabilities) may reject the call. |
| 03–05 | – | Unassigned. |
| 06 | – | Station Level Rating The “06” digit pair is used when the customer has subscribed to a class of service in order to be provided with real time billing information. For example, hotel/motels, served by PBXs, receive detailed billing information, including the calling party's room number. When the originating switching system does not receive the detailed billing information, e.g., room number, this “06” code allows the call to be routed to an operator or operator services system to obtain complete billing information. The rating and/or billing information is then provided to the service subscriber. This code is used only when the directory number (DN) is not accompanied by an automatic room/account identification. |
| 07 | – | Special Operator Handling Required - calls generated from stations that require further operator or Operator Services System screening are accompanied by this “07” code. The code is used to route the call to an operator or Operator Services System for further screening and to determine if the station has a denied-originating class of service or special routing/billing procedures. If the call is unauthorized, the calling party will be routed to a standard intercept message. |
| 08–09 | – | Unassigned. |
| 10 | – | Not assignable - conflict with 10X test code |
| 11 | – | Unassigned. |
| 12–19 | – | Not assignable - conflict with international outpulsing code |
| 20 | – | Automatic Identified Outward Dialing (AIOD) - without AIOD, the billing number for a PBX is the same as the PBX Directory Number (DN). With the AIOD feature, the originating line number within the PBX is provided for charging purposes. If the AIOD number is available when ANI is transmitted, code “00” is sent. If not, the PBX DN is sent with ANI code “20”. In either case, the AIOD number is included in the AMA record. |
| 21–22 | – | Unassigned. |
| 23 | – | Coin or Non-Coin - on calls using database access, e.g., 800, ANI II 23 is used to indicate that the coin/non-coin status of the originating line cannot be positively distinguished for ANI purposes by the SSP. The ANI II pair 23 is substituted for the II pairs that would otherwise indicate that the non-coin status is known, such as 00, or when there is ANI failure. ANI II 23 may be substituted for a valid 2-digit ANI pair on 0-800 calls. In all other cases, ANI II 23 should not be substituted for a valid 2-digit ANI II pair that is forward to an SSP from an EAEO. Some of the situations in which the ANI II 23 may be sent: - Calls from non-conforming end offices (CAMA or LAMA types) with combined coin/non-coin trunk groups. - Calls - Type 1 Cellular Calls - Calls from PBX Trunks - Calls from Centrex Tie Lines |
| 24 | – | Code 24 identifies a toll free service call that has been translated to a Plain Old Telephone Service (POTS) routable number via the toll free database that originated for any non-pay station. If the received toll free number is not converted to a POTS number, the database returns the received ANI code along with the received toll free number. Thus, this 24 code indicates that this is a toll free service call since that fact can no longer be recognized simply by examining the called address. |
| 25 | - | Code 25 identifies a toll free service call that has been translated to a Plain Old Telephone Service (POTS) routable number via the toll free database that originated from any pay station, including inmate telephone service. Specifically, ANI II digits 27, 29, and 70 will be replaced with Code 25 under the above stated conditions. |
| 26 | – | Unassigned. |
| 27 | – | Code 27 identifies a line connected to a pay station that uses network provided coin control signaling. II 27 is used to identify this type of pay station line irrespective of whether the pay station is provided by a LEC or non-LEC. II 27 is transmitted from the originating end office on all calls made from these lines |
| 28 | – | Unassigned. |
| 29 | – | Prison/Inmate Service - the ANI II digit pair 29 is used to designate lines within a confinement/detention facility that are intended for inmate/detainee use and require outward call screening and restriction (e.g., 0+ collect only service). A confinement/detention facility may be defined as including, but not limited to, Federal, State and/or Local prisons, juvenile facilities, immigration and naturalization confinement/detention facilities, etc., which are under the administration of Federal, State, City, County, or other Governmental agencies. Prison/Inmate Service lines will be identified by the customer requesting such call screening and restriction. In those cases where private paystations are located in confinement/detention facilities, and the same call restrictions applicable to Prison/Inmate Service are required, the ANI II digit for Prison/Inmate Service will apply if the line is identified for Prison/Inmate Service by the customer. |
| 30–32 | – | Intercept - where the capability is provided to route intercept calls (either directly or after an announcement recycle) to an access tandem with an associated Operator Services System, the following ANI codes should be used: PD 30 - Intercept (blank) - for calls to unassigned directory number (DN) PD 31 - Intercept (trouble) - for calls to directory numbers (DN) that have been manually placed in trouble-busy state by telephone company personnel PD 32 - Intercept (regular) - for calls to recently changed or disconnected numbers |
| 33 | – | Unassigned. |
| 34 | – | Telco Operator Handled Call - after the Telco Operator Services System has handled a call for an IC, it may change the standard ANI digits to “34”, before outpulsing the sequence to the IC, when the Telco performs all call handling functions, e.g., billing. The code tells the IC that the BOC has performed billing on the call and the IC only has to complete the call. |
| 35-39 | – | Unassigned. |
| 40–49 | – | Unrestricted Use - locally determined by carrier |
| 50–51 | – | Unassigned. |
| 52 | – | Outward Wide Area Telecommunications Service (OUTWATS) - this service allows customers to make calls to a certain zone(s) or band(s) on a direct dialed basis for a flat monthly charge or for a charge based on accumulated usage. OUTWATS lines can dial station-to-station calls directly to points within the selected band(s) or zone(s). The LEC performs a screening function to determine the correct charging and routing for OUTWATS calls based on the customer's class of service and the service area of the call party. When these calls are routed to the interexchange carrier via a combined WATS-POTS trunk group, it is necessary to identify the WATS calls with the ANI code “52”. |
| 53–59 | – | Unassigned. |
| 60 | – | TRS - ANI II digit pair 60 indicates that the associated call is a Telecommunications Relay Service (TRS) call (for the hearing impaired) delivered to a transport carrier from a TRS Provider and that the call originated from an unrestricted line (i.e., a line for which there are no billing restrictions). Accordingly, if no request for alternate billing is made, the call will be billed to the calling line. |
| 61 | – | Cellular/Wireless PCS (Type 1) - The “61” digit pair is to be forwarded to the interexchange carrier by the local exchange carrier for traffic originating from a cellular/wireless PCS carrier over type 1 trunks. (Note: ANI information accompanying digit pair “61” identifies only the originating cellular/wireless PCS system, not the mobile directory placing the call.) |
| 62 | – | Cellular/Wireless PCS (Type 2) - The “62” digit pair is to be forwarded to the interexchange carrier by the cellular/wireless PCS carrier when routing traffic over type 2 trunks through the local exchange carrier access tandem for delivery to the interexchange carrier. (Note: ANI information accompanying digit pair “62” identifies the mobile directory number placing the call but does not necessarily identify the true call point of origin.) |
| 63 | – | Cellular/Wireless PCS (Roaming) - The “63” digit pair is to be forwarded to the interexchange carrier by the cellular/wireless PCS subscriber “roaming” in another cellular/wireless PCS network, over type 2 trunks through the local exchange carrier access tandem for delivery to the interexchange carrier. (Note: Use of “63” signifies that the “called number” is used only for network routing and should not be disclosed to the cellular/wireless PCS subscriber. Also, ANI information accompanying digit pair “63” identifies the mobile directory number forwarding the call but does not necessarily identify the true forwarded-call point of origin.) |
| 64–65 | – | Unassigned. |
| 66 | – | TRS - ANI II digit pair 66 indicates that the associated call is a TRS call delivered to a transport carrier from a TRS Provider, and that the call originates from a hotel/motel. The transport carrier can use this indication, along with other information (e.g. whether the call was dialed 1+ or 0+) to determine the appropriate billing arrangement (i.e., bill to room or alternate bill). |
| 67 | – | TRS - ANI II digit pair 67 indicates that the associated call is a TRS call delivered to a transport carrier from a TRS Provider and that the call originated from a restricted line. Accordingly, sent paid calls should not be allowed and additional screening, if available, should be performed to determine the specific restrictions and type of alternate billing permitted. |
| 68–69 | – | Unassigned. |
| 70 | – | Private Paystations - Identifies a line connected to a pay station (including both coin and coinless stations), which does not use network provided coin control signaling. II 70 is used to identify this type pay station line irrespective of whether the pay station is provided by a LEC or a non-LEC. II 70 is transmitted from the originating end office on all calls made from these lines. |
| 71–79 | – | Unassigned. |
| 80–89 | – | Reserved for Future Expansion “to” 3-digit Code |
| 90–92 | – | Unassigned. |
| 93 | – | Access for private virtual network types of service: the ANI code “93” indicates, to the IC, that the originating call is a private virtual network type of service call. |
| 94 | – | Unassigned. |
| 95 | – | Unassigned - conflict with Test Codes 958 and 959 |
| 96–99 | – | Unassigned |

## 1.9	Toll-free (e.g. 800) and 900 NXX Code Assignments, Routing Status, Definitions

800 (Toll Free) (also pertains to 888, 877, etc.):

Prior to 1993, SAC 800 (i.e., toll-free) NXX’s were assigned to entities under an interim plan to permit development of an SMS-800-Database that would provide service-provider number portability.  After development was completed, 800 NXX’s were moved to the database over a period of time.  Currently, all Toll Free NXX’s are in the SMS-800-Database except the following (note that *XX refers to XX=22 through 88, as such become activated):

| ASSIGNED TO NON-US CARIBBEAN |  |  |  |
| --- | --- | --- | --- |
| SAC | NXX | Company Assignment | Country |
| 800 | 271 | Textel | Trinidad |
| 800 | 389 | Batelco | Bahamas |
| 800 | 415 | All American Cables | Dominican Republic |
| 800 | 534 | Bartelco | Barbados |
| 800 | 623 | Bermuda Tel Co | Bermuda |
| 800 | 703 |  | For Future Assignment |
| 800 | 740 | STSJ Telephone Co | Virgin Islands (in Non-US areas) |
| 800 | 744 | Cable & Wireless |  |
| 800 | 751 | Codetel | Dominican Republic |
| 800 | 904 |  | For Future Assignment |
| 800 | 907 | Tricom | Dominican Republic |
| OTHER |  |  |  |
| SAC | NXX | Reason | Comment |
| 8XX | 555 | Not available or assignment | 800-555 is used for “Toll-free” Directory Assistance |
| 8XX | 911 | Reserved-use code |  |
| 800 | 855 | Hearing impaired |  |
| 800 | N02 | Radio Common Carrier use | N=2-9 |
| 800 | N12 | Radio Common Carrier use | N=2-9 |

There is no SAC “800” data in the LERG Routing Guide.  The numbers not listed above are considered service-provider portable and are in the 800-SMS -Database.  That is, any line number in NXX’s not listed above are individually obtained and serviced by any sanctioned 800-service provider (one line number may be serviced by one provider, the next number by another).  All numbers in any relief for 800 (888, 877, 866, etc.) are considered toll-free and are a component of the “800-SMS-Database” addressed herein.  Questions about the numbers listed above should be directed to the North American Numbering Plan Administration (www.nanpa.com).  Questions about the numbers in the 800-SMS-Database should be addressed to the Number Administration Service Center (NASC) at 888-767-3300

900 (information provider – pay services):

The following provides information regarding the data associated with non-geographic Personal Communications Services NXX codes (NPA 500) listed in Volume 8, Section 4.9, and LERG6 data files.

The information provided below explains how the NPA 900 NXX codes are represented in the LERG.  Items noted with a “*” are defaulted values, as the field does not technically apply:

| ###### FIELD NAME | ###### VALUE OR INTERPRETATION |
| --- | --- |
| LATA | 999* |
| LATA Name | RESERVED FOR SVCS* |
| STATUS | No difference from standard definition |
| EFFDATE (Effective Date) | This is the assignment date of the NXX. Activation is not implied and may be pursuant to tariffs. |
| NPA | 900 |
| NXX (COC) | Specific NXX’s that are assigned by NANPA |
| BLOCK | A |
| COC TYPE | EOC* |
| SSC | N* |
| OCN | “OCN” (Operating Company Number) is used to identify the party assigned the NPA 900 NXX. See OCN in the Glossary section of this document for more information |
| AOCN | TRA1 |
| LOC NAME | SACSERVICE* |
| LOC COUNTY | Blank* |
| LOC STATE | XX* |
| RATE CENTER / RCTYPE | SACSERVICE* / Blank* |
| SWITCH | (see explanation following this table) |
| DIND | Y |
| TRDIG EO | 7* |
| TRDIG AT | 10* |
| PORTABLE | N |
| SHA IND | 00* |
| TEST LINE information | Blank* |
| LINE RANGE | 0000-9999 |
| POOLING INDICATOR | N |

The SWITCH field provides some information about the 900 NXX code assignee, such as the Carrier Identification Code (CIC), or the assignment to a LEC or other company.  The SWICTH field value in this case is not a CLLI code.

SWITCH Example	Explanation

ASGNIC0222		0222 = Carrier Identification (CIC) Code

INTRALAT722		LATA served by the NXX = 722

SWCHxxUNKNO	Switch unknown, assigned in state, etc. defined by xx (See STATE)

TELECOMMCAN	Intra-Canada Routing Only

1.10	500 NXX Personal Communications Services (PCS) Code Assignments, Definitions

The following provides information regarding the data associated with non-geographic Personal Communications Services NXX codes (NPA 500) listed in Volume 8, Section 4.9, and LERG6 data files.

Questions regarding these assignments should be referred to the NANPA website, www.nanpa.com or:

NANPA

46000 Center Oak Plaza

Sterling, VA  20166

The information provided below explains how the NPA 500 NXX codes are represented in the LERG.  Items noted with a “*” are defaulted values, as the field does not technically apply:

| ###### FIELD NAME | ###### VALUE OR INTERPRETATION |
| --- | --- |
| LATA | 999* |
| LATA Name | RESERVED FOR SVCS* |
| STATUS | No difference from standard definition |
| EFFDATE (Effective Date) | This is the assignment date of the NXX. Activation is not implied and may be pursuant to tariffs. |
| NPA | 500 |
| NXX (COC) | Specific NXX’s that are assigned by NANPA |
| BLOCK | A |
| COC TYPE | EOC* |
| SSC | N* |
| OCN | “OCN” (Operating Company Number) is used to identify the party assigned the NPA 500 NXX. See OCN in the Glossary section of this document for more information |
| AOCN | TRA1 |
| LOC NAME | SACSERVICE* |
| LOC COUNTY | Blank* |
| LOC STATE | XX* |
| RATE CENTER / RCTYPE | SACSERVICE* / Blank* |
| SWITCH | (see explanation following this table) |
| DIND | Y |
| TRDIG EO | 7* |
| TRDIG AT | 10* |
| PORTABLE | N |
| SHA IND | 00* |
| TEST LINE information | Blank* |
| LINE RANGE | 0000-9999 |
| POOLING INDICATOR | N |

Wireless companies who want a PCS NXX must have a CIC assigned to them because LECs must send a 0ZZ and CIC to identify the company.

The SWITCH field provides some information about the 500 NXX code assignee, such as the Carrier Identification Code (CIC), or the assignment to a LEC or other company.  The SWICTH field value in this case is not a CLLI code.

SWITCH Example	Explanation

ASGNPC0222		0222 = Carrier Identification (CIC) Code

ASGNPCSOCNX	OCNX = Refer to OCN (i.e. LERG1) for OCN contact

- 1.11	710 NXX Code Assignments, Routing Procedures, Definitions

SC 710 is a non-geographic, toll free Numbering Plan Area (NPA) code that has been assigned by the North American Numbering Plan Administration (NANPA) to the United States Government.  The NXX codes within SC 710 are administered by the U.S. Government instead of by the NANPA Code Administrators.  Questions concerning the assignment of 710-NXX codes should be addressed to

The Office of the Manager National Communications System (OMNCS)

GETS Technical Director

Telephone:  703-607-4800

This section describes how routing data for SC 710 NXX’s are provided in the LERG Routing Guide and other Telcordia Routing Administration data products that contain these fields.  The NXX’s will be in LATA 999.

When assigned by the OMNCS, the 710 NXX’s listed in Volume 8, Section 4.9 and in LERG6 data files.  Each data field in LERG6 is described below as applies to NXX’s assigned in NPA 710.  Items noted with a “*” are defaulted values, as the field does not technically apply:

| ###### FIELD NAME | ###### VALUE OR INTERPRETATION |
| --- | --- |
| LATA | 999* |
| LATA Name | RESERVED FOR SVCS* |
| STATUS | No difference from standard definition |
| EFFDATE (Effective Date) | This is the assignment date of the NXX. Activation is not implied and may be pursuant to tariffs. |
| NPA | 710 |
| NXX (COC) | Specific NXX’s that are assigned by OMNCS |
| BLOCK | A |
| COC TYPE | EOC* |
| SSC | N* |
| OCN | 4758 is the OCN assigned to the U.S. Government |
| AOCN | 0341 (Sprint Local Tel. Div.) |
| LOC NAME | 710SERVICE* |
| LOC COUNTY | Blank |
| LOC STATE | TS (i.e., correlates to value in 5-6th position of SWITCH) |
| RATE CENTER / RCTYPE | 710SERVICE* / Blank* |
| SWITCH | RTGETSAGENT (see explanation following this table) |
| DIND | Y |
| TRDIG EO | 10* |
| TRDIG AT | 10* |
| PORTABLE | N |
| SHA IND | 00* |
| TEST LINE information | Blank* |
| LINE RANGE | 0000-9999 |
| POOLING INDICATOR | N |

SWITCH value of “RTGETSAGENT”:

In some cases, the U.S. Government has special arrangements with local carriers to handle calls to SC 710 NXX’s.  In other cases, where the local carrier, cellular/PCS carrier, PBX operator, payphone operator or any other call originator does not have a special arrangement with the U.S. Government, the call originator should “Route To the Government Emergency Telecommunications Service (GETS) AGENT” (RTGETSAGENT).  The agents to which calls should be routed in these cases currently are AT&T, Sprint, or MCI WorldCom.  Payphone operators can receive compensation under the same provisions as for toll free calling using SAC 800, 888, 877, 866, 855, or other toll-free SACs.

## Country Codes

For the paper LERG Routing Guide see Section 3.5, for the LERG Routing Guide data files see LERG2.DAT.

- The “Notes” associated with the assignment of Country Codes are listed below and are provided for your reference.

The code assignments appearing in the LERG are derived from the most recent ITU (International Telecommunication Union) Bulletin as well as any additional ITU Bulletins issued to announce specific Country Code assignment changes.  The assigned codes are taken from the “List of ITU-T Recommendation E.164 Assigned Country Codes” (http://www.itu.int/bulletin/index.html) web site.

It is recognized that the ITU assigns codes and that international carriers cooperate to activate codes.  Notifications during the interim period will refer to (1) assignments and (2) testing and activation dates.  The material used in the LERG is reproduced with the prior authorization of the ITU as the copyright holder; its presentation in the LERG is the sole responsibility of Telcordia Technologies and does not engage the responsibility of the ITU.

The assignments are being provided so that service providers will have a list of all countries that might be dialed by their customers.

Questions about ordering English-language publications of the ITU should be directed to:

- Phillips Business Information

1201 Seven Locks Road

Potomac, MD 20845

TEL: 1-800-777-5006

FAX: 301-309-3847

Questions about the contents of any ITU publications should be directed to:

- International Telecommunications Union

General Secretariat - Sales Section

Place des Nations

CH1211 Geneva 20

SWITZERLAND

TEL: 41 22 730 51 11

FAX: 41 22 733 72 56

- NOTES

The following footnotes correspond to the associated records listed in the Country Code Section of the paper LERG Routing Guide (Section 3.5) and the LERG2 data files.  Any additions or corrections are based on the publication of the ITU-T Telecommunication Standardization Section of ITU “List of ITU-T Recommendation E.164 Assigned Country Codes (Position in February 2004).”

- Integrated numbering area.

- Code shared between Mayotte Island and Comoros (Union of the).

- Will be allocated, only after all three digit codes from groups of ten are exhausted.

- Associated with shared country code 878 for Universal personal telecommunications (UPT), the identification code 10 has been assigned to the network of VISIONng, +878 10.

- Reserved for future use.

- Including Australian Antarctic Territory and Norfolk Island.

- U.A.E.: Abu Dhabi, Ajman, Dubai, Fujeirah, Ras Al Khaimah, Sharjah, Umm Al Qiwain.

- Including Christmas Island and Cocos-Keeling Islands.

- Associated with shared country code 882, the following two-digit identification code reservations or assignments have been made for the international networks of:

| APPLICANT | NETWORK | COUNTRY CODE / IDENTIFICATION CODE | STATUS | DATE |
| --- | --- | --- | --- | --- |
| British Telecommunications plc | Global Office Application | +882 10 | Assigned | 12/1/98 |
| MCI/WorldCom | HyperStream International (HIS) Data Network | +882 12 | Assigned | 5/10/99 |
| Telespazio S.p.A. | EMS Regional Mobile Satellite Systems | +882 13 | Assigned | 6/11/98 |
| Verizon | GTE ATM Networks | +882 14 | Assigned | 02/01/04 |
| Reach | Global International ATM Network | +882 15 | Assigned | 02/01/04 |
| United Arab Emirates Administration | Thuraya RMSS Network | +882 16 | Assigned | 06/01/00 |
| Asia Cellular Satellite (ACeS) | Garuda Mobile Telecommunication Satellite System | +882 20 | Assigned | 02/01/04 |
| Cable & Wireless plc | Cable & Wireless Global Network | +882 22 | Assigned | 12/01/98 |
| Sita-Equant Joint Venture | Sita-Equant Network | +882 23 | Assigned | 02/01/04 |
| TeliaSonera AB | TeliaSonera multinational ATM Network | +882 24 | Assigned | 02/01/04 |
| Deutsche Telekom | Deutsche Telekom multinational ATM network | +882 28 | Assigned | 02/01/04 |
| Q-Tel (NZ) Ltd. (ex-World IT) | SkyMessenger International Network | +882 29 | Assigned | 02/01/04 |
| Singapore Telecomm | Global Reach Network | +882 30 | Reserved | 02/01/04 |
| Telekom Malaysia | Global International ATM Network | +882 31 | Assigned | 02/01/04 |
| Maritime Communications Partner A.S. | MCP network | +882 32 | Assigned | 02/01/04 |
| Oration Technologies, Inc. | Oration Technologies Network | +882 33 | Assigned | 02/01/04 |
| Global Networks, Inc. | Global Networks | +882 34 | Assigned | 02/01/04 |

- Associated with shared country code 881, the following one-digit identification codes have been made for the GMSS networks:

| NETWORK | COUNTRY CODE / IDENTIFICATION CODE | STATUS |
| --- | --- | --- |
| ICO Global Communications | +811 0 and +811 1 | Reserved |
| Ellipso | +881 2 and +881 3 | Assigned |
| Iridium Satellite LLC | +881 6 and +881 7 | Assigned |
| Globalstar | +881 8 and +881 9 | Assigned |

- Reserved for the Palestinian Authority.

- Reserved for E.164 country code expansion.

- Associated with shared country code 388 for Group of countries, the following one-digit identification code has been assigned to the network ETNS (European Telephone Numbering Space) 388 3.

- Within People's Republic of China Country Code 86, Country Code 866 has been assigned to Taiwan.  This is documented in Notification No. 1157 of the International Telecommunications Union (ITU) and is the only code that is internationally recognized by all members of the ITU.

- The code 886 is the code self-attributed by Taiwan and is not in any way internationally recognized by the ITU.  The use by Taiwan of the self-attributed code 886 has, up to now, not caused problems with regard to communications of other Members of the ITU since there have been sufficient codes available to satisfy incoming requests without the ITU having to attribute code 886.

## 1.13	976-LIKE Codes

976-LIKE codes are NXX’s that function as NXX 976 codes.  976 has been used essentially as an NXX for pay services provided by various service providers (e.g. lottery numbers, time, weather, etc.).  Use may vary by NPA.  In response to consumer issues, regulatory decisions, etc. (all of which may vary by state and company) non-976 NXX’s have been established in some jurisdictions to function as 976.  These 976-LIKE NXX’s may be used to isolate certain types of services from those provided via 976.  These NXX’s are routed to the service providers’ processes (not necessarily a specific switch) and are usually IntraLATA in nature.  Such codes may be found in the LERG Routing Guide as a type of “Oddball” code and should carry the COCTYPE value of INP (Information Provider), although all INP coded records are not necessarily “976-Like” situations.

## 1.14	Telcordia® Routing Administration Product Ordering Information

The LERG Routing Guide is produced quarterly in book format (Volumes 1 through 9).  The LERG Routing Guide is also available monthly as data versions a CD ROM (also downloadable from the web), or transmitted (without the Microsoft® Access database version) via NDM (Network Data Mover); each of these data versions contains the data in ALL nine volumes of the paper LERG.  If you are interested in ordering the LERG (paper is no longer offered to new subscribers), further information and an order form can be obtained at www.trainfo.com.  Alternatively, you may call the Telcordia® Routing Administration Customer Service Center on 866-NPA-NXXS (866-672-6997) or 732-699-6700 to obtain further information.

The LERG Routing Guide is a licensed data product and is provided under a specific enterprise license agreement that defines how it can be used in a given company.  Additional information on licensing of the LERG Routing Guide can be found at www.trainfo.com, or by calling the Telcordia® Routing Administration Customer Service Center.

COMMON LANGUAGE® is a registered trademark and CLLI™ is a trademark of Telcordia Technologies

COMMON LANGUAGE is a registered trademark of Telcordia Technologies

COMMON LANGUAGE is a registered trademark and CLLI is a trademark of Telcordia Technologies

Vertical Service codes in the *1X range will be assigned only after all other available *XX codes have been assigned, i.e., *0X and *4X through *93.